Tender No:. 2024-25/Estate/2B/LNM-866

The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non-Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposals from competent vendors for “Annual Routine Maintenance Work for Fire Fighting and Alarm System”. Bidders can submit their sealed offers/s at the following address by Email/Speed-Post/ Registered Post/ By-hand/courier before 05:00 PM on 25 January 2025.

Filing of Tender:

Option 1: Physical submission of offers (hard copy) in sealed envelopes.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the institute in person/ Speed-Post/ Registered Post/ By-hand/courier of the Institute after marking due entries in the Receipt Book available with the Security Officer at the main gate.

The tender subject and reference number must be mentioned on the envelope.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Email: directoroffice@lnmiit.ac.in

Option 2: Password-protected online submission via Email

Password-protected offers can be submitted by the bidders via email at the below-mentioned email IDs.

tender-document@lnmiit.ac.in (password-protected document)

tender-password@lnmiit.ac.in (password only)

The tender subject and reference number must be mentioned in the subject line of the emails.

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory, are required to be submitted in two separate envelopes/emails.

Envelopes/Emails should be duly marked as follows:

- Technical bid for the “Annual Routine Maintenance Work for Fire Fighting and Alarm System”.

- Commercial Bid for the “Annual Routine Maintenance Work for Fire Fighting and Alarm System”.

Both Envelopes/files should be enclosed/attached in a single envelope/email bearing the heading “Bid for the “Annual Routine Maintenance Work for Fire Fighting and Alarm System (Tender No. : 2024-25/Estate/2B/LNM-866) ”.

Bids are liable to be rejected:

- If the above-mentioned bid submission procedure is not followed.

- Incomplete and non-conforming bids.

- Arbitration: All disputes and differences arising between the two parties in connection with this order shall be mutually settled as far as possible, failing which, all such disputes shall be referred to arbitration and settlement under the provision of the Arbitration and Conciliation Act, 1996 and as amended from time to time or any such law for the time being prevailing in India. The venue of such arbitration shall be Jaipur.

Technical Bid: (in a separate sealed envelope/via email)

# Bid for Annual Routine Maintenance Work for Fire Fighting and Alarm System (Tender No. : 2024-25/Estate/2B/LNM-866)

While submitting quotations, the following procedure/ formalities and conditions may kindly be adhered.

- Company profile or brief note about the bidder's company, PAN number, and GST number should be submitted by the Vendor.

- The List of Directors/Partners/Proprietor/Members of the Firm to be attached.

- Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members to be provided.

- Turnover for the last three years to be mentioned. (Copy to be enclosed)

- List of the client to whom the vendor has delivered the goods or provided the services in the past.

- Authorized Supplier Certificate

- Specification as mentioned below, shall be provided as per the details given below:

## Scope of work

- The scope of work will cover the supply/service/SITC of given items at LNMIIT Jaipur campus.

- The Vendor is required to submit the complete documentation of the supplied item after delivery.

- SITE CONDITION:

Before quoting, the contractor must visit the site to inspect the work and fully acquaint himself with the conditions and scope of work with regards to accessibility of site required for the satisfactory execution of work. No compensation on account of any site difficulties will be entertained later, after the award of the works.

Technical specification:

| S. No. | DESCRIPTION OF ITEM (S) | UNIT | QTY |
| --- | --- | --- | --- |
| (A) | Fire Fighting System |  |  |
| 1 | SITC of Air Vessel |  |  |
|  | Supplying, Installation, Testing and Commissioning of air cushion tank (Air vessel) 200mm dia. & 1500 mm height having 8 mm thick dished ends made of 8mm thick M.S.Pipe, and providing 25mm dia. with brass air release valve, Safety valve, Pressure gauge, suitable M.S.support and minor civil works etc. as required. (Location _ Bh-3 & 4 , Girls Hostels, Studio Apartments, Academic Block ) | Nos. | 6 |
| 2 | SITC of Pressure Gauge |  |  |
|  | Providing, fixing, testing & commissioning of 100 mm dia Bourdon type, Stainless Steel dial type pressure gauge including brass isolation valve and pipe having calibration of 0-15 Kg/cm2. ( Make : Hguru, / Fiebig ) | Nos. | 4 |
| 3 | SITC of Fire Hose |  |  |
|  | Supplying, Installation, Testing and Commissioning 100% synthetic flax canvas Non-percolating FIRE hose I.S.I marked 63mm diax15m long with Gun metal male & female couplings (ISI marked) bound & riveted to hose pipes with SS rivets and SS wire as required. (MAKE- SAFEGUARD/ PADMINI/GUARDS/OMEX/FALCON) | Nos | 60 |
| 4 | SITC of Short Branch Pipe |  |  |
|  | Supplying, Installation, Testing and Commissioning Gun metal Stainless Steel 63mm dia. Short Branch Pipe (ISI marked) with 20mm dia nozzle as required. (MAKE- SAFEGUARD/GEETECH/GUARDS/OMEX/FALCON) | Nos | 15 |
| 5 | SITC of Single Headed G.M. Oblique Hydrant Landing Valve |  |  |
|  | Supplying, installation, testing and commissioning of single headed Gun metal Stainless Steel, ISI marked oblique hydrant landing valve with 80mm dia flanged inlet & 63mm dia instanteous type female outlet complete with A.B.S. plastic cap and chain, twist release type lug and all accessories complete as required. (MAKE- SAFEGUARD/GEETECH/GUARDS/FALCON) | Nos | 8 |
| 6 | SITC of Butterfly Valve / Gate valve |  |  |
|  | Providing and fixing of cast iron double flanged Butterfly Valve, wafer type PN1.6 IS:13095 complete with nuts and bolts rubber insertion all as required. (MAKE- UTTAM/ CASTLE/KARTAR/ ZOLOTO ) |  |  |
| a) | 250mm dia. | Nos. | 2 |
| b) | 150mm dia | Nos. | 10 |
| c) | 100 mm dia | Nos. | 2 |
| d) | 80 mm dia | Nos. | 2 |
| 7 | SITC of Cast Iron Flanged Non Return Valve |  |  |
|  | Providing and fixing of cast iron double flanged type non-return valve PN 1.6 IS:13095 complete with nut and bolts rubber insertion as required: (Wafer type) (MAKE- UTTAM/ CASTLE/KARTAR) |  |  |
| a) | 150mm dia | Nos. | 4 |
| b) | 100mm dia | Nos. | 4 |
| c) | 80 mm dia | Nos. | 1 |
| 8 | SITC of EXPANSION ELBOWS |  |  |
|  | 150 mm dia | Nos | 1 |
|  | 100 mm dia. | Nos | 1 |
|  | 80 mm dia, | Nos | 1 |
| 9 | SITC of Ball valves 25 mm dia Ball Valves | Nos. | 5 |
| 10 | Painting with proper cleaning of complete Existing Fire Pipe line ( Above Ground ) including painting of pipes and fittings with red paint over a coat of ready mixed primer, both of approved quality | Mtrs | 600 |
| (B) | Servicing Conventional Fire Alarm System Panels |  |  |
| 1 | Servicing of conventional type Photo electric type Smoke Detector with back box and original base. | Nos | 300 |
| 2 | Servicing of Manual Call Points complete with MS / ABS body, LED indicator,etc. | Nos | 200 |
| 3 | Servicing of 24 volt operated Sounder cum flasher complete | Nos. | 50 |
| 4 | Servicing of Microprocessor based 2/4/8 zone Fire Alarm Control Panels with 2 nos 12V 7AH battrey & inbuilt battrey charger complete as required. | Nos | 10 |
| 5 | Supply & fixing of Photolouminiscent type EXIT signages on 3 mm acrylic sheet of size 12 x 6 inch at various location complete with fixing arangement of waal / ceiling as required | Nos | 85 |
| 6 | Providing & fixing required FIRE Safety & Industrial safety charts as per the requirement of Fire & safety department | Nos | 4 |
| ( C ) | Painting with Proper Cleaning of Fire Buckets |  |  |
| 1 | Painting with proper cleaning of complete of Fire Buckets 4 nos with round botom made out of MS / GI sheet painted with Fire red colour complete with Fire bucket stand having hanging capacity of 4 buckets | Set | 30 |
| 2. | Preparation of required set of drawings for submission to state Govt / Fire department for taking NOC , Job include the necessary liasioning work with the state Fire services / Municipal office complete as required, NOC renewal work include Old Campus & . |  |  |

Commercial Bid: (in separate sealed envelope/password protected file in Email)

# Bid for

# Annual Routine Maintenance Work for Fire Fighting and Alarm System (Tender No. : 2024-25/Estate/2B/LNM-866)

| S. No. | DESCRIPTION OF ITEM (S) | UNIT | QTY | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| (A) | Fire Fighting System |  |  |  |  |
| 1 | SITC of Air Vessel |  |  |  |  |
|  | Supplying, Installation, Testing and Commissioning of air cushion tank (Air vessel) 200mm dia. & 1500 mm height having 8 mm thick dished ends made of 8mm thick M.S.Pipe, and providing 25mm dia. with brass air release valve, Safety valve, Pressure gauge, suitable M.S.support and minor civil works etc. as required. (Location _ Bh-3 & 4 , Girls Hostels, Studio Apartments, Academic Block ) | Nos. | 6 |  |  |
| 2 | SITC of Pressure Gauge |  |  |  |  |
|  | Providing, fixing, testing & commissioning of 100 mm dia Bourdon type, Stainless Steel dial type pressure gauge including brass isolation valve and pipe having calibration of 0-15 Kg/cm2. ( Make : Hguru, / Fiebig ) | Nos. | 4 |  |  |
| 3 | SITC of Fire Hose |  |  |  |  |
|  | Supplying, Installation, Testing and Commissioning 100% synthetic flax canvas Non-percolating FIRE hose I.S.I marked 63mm diax15m long with Gun metal male & female couplings (ISI marked) bound & riveted to hose pipes with SS rivets and SS wire as required. (MAKE- SAFEGUARD/ PADMINI/GUARDS/OMEX/FALCON) | Nos | 60 |  |  |
| 4 | SITC of Short Branch Pipe |  |  |  |  |
|  | Supplying, Installation, Testing and Commissioning Gun metal Stainless Steel 63mm dia. Short Branch Pipe (ISI marked) with 20mm dia nozzle as required. (MAKE- SAFEGUARD/GEETECH/GUARDS/OMEX/FALCON) | Nos | 15 |  |  |
| 5 | SITC of Single Headed G.M. Oblique Hydrant Landing Valve |  |  |  |  |
|  | Supplying, installation, testing and commissioning of single headed Gun metal Stainless Steel, ISI marked oblique hydrant landing valve with 80mm dia flanged inlet & 63mm dia instanteous type female outlet complete with A.B.S. plastic cap and chain, twist release type lug and all accessories complete as required. (MAKE- SAFEGUARD/GEETECH/GUARDS/FALCON) | Nos | 8 |  |  |
| 6 | SITC of Butterfly Valve / Gate valve |  |  |  |  |
|  | Providing and fixing of cast iron double flanged Butterfly Valve, wafer type PN1.6 IS:13095 complete with nuts and bolts rubber insertion all as required. (MAKE- UTTAM/ CASTLE/KARTAR/ ZOLOTO ) |  |  |  |  |
| a) | 250mm dia. | Nos. | 2 |  |  |
| b) | 150mm dia | Nos. | 10 |  |  |
| c) | 100 mm dia | Nos. | 2 |  |  |
| d) | 80 mm dia | Nos. | 2 |  |  |
| 7 | SITC of Cast Iron Flanged Non Return Valve |  |  |  |  |
|  | Providing and fixing of cast iron double flanged type non-return valve PN 1.6 IS:13095 complete with nut and bolts rubber insertion as required: (Wafer type) (MAKE- UTTAM/ CASTLE/KARTAR) |  |  |  |  |
| a) | 150mm dia | Nos. | 4 |  |  |
| b) | 100mm dia | Nos. | 4 |  |  |
| c) | 80 mm dia | Nos. | 1 |  |  |
| 8 | SITC of EXPANSION ELBOWS |  |  |  |  |
|  | 150 mm dia | Nos | 1 |  |  |
|  | 100 mm dia. | Nos | 1 |  |  |
|  | 80 mm dia, | Nos | 1 |  |  |
| 9 | SITC of Ball valves 25 mm dia Ball Valves | Nos. | 5 |  |  |
| 10 | Painting with proper cleaning of complete Existing Fire Pipe line ( Above Ground ) including painting of pipes and fittings with red paint over a coat of ready mixed primer, both of approved quality | Mtrs | 600 |  |  |
| (B) | Servicing Conventional Fire Alarm System Panels |  |  |  |  |
| 1 | Servicing of conventional type Photo electric type Smoke Detector with back box and original base. | Nos | 300 |  |  |
| 2 | Servicing of Manual Call Points complete with MS / ABS body, LED indicator,etc. | Nos | 200 |  |  |
| 3 | Servicing of 24 volt operated Sounder cum flasher complete | Nos. | 50 |  |  |
| 4 | Servicing of Microprocessor based 2/4/8 zone Fire Alarm Control Panels with 2 nos 12V 7AH battrey & inbuilt battrey charger complete as required. | Nos | 10 |  |  |
| 5 | Supply & fixing of Photolouminiscent type EXIT signages on 3 mm acrylic sheet of size 12 x 6 inch at various location complete with fixing arangement of waal / ceiling as required | Nos | 85 |  |  |
| 6 | Providing & fixing required FIRE Safety & Industrial safety charts as per the requirement of Fire & safety department | Nos | 4 |  |  |
| ( C ) | Painting with Proper Cleaning of Fire Buckets |  |  |  |  |
| 1 | Painting with proper cleaning of complete of Fire Buckets 4 nos with round botom made out of MS / GI sheet painted with Fire red colour complete with Fire bucket stand having hanging capacity of 4 buckets | Set | 30 |  |  |
| 2 | Preparation of required set of drawings for submission to state Govt / Fire department for taking NOC , Job includes the necessary liasioning work with the state Fire services / Municipal office complete as required, Online Application Charges for NOC Renewal, NOC renewal work Old Campus include. | Job | 1 |  |  |

| Amount |  |
| --- | --- |
| GST |  |
| Total |  |

- Taxes like GST shall be clearly mentioned. If nothing is mentioned, these charges shall be deemed included.

- If the supplier does not have a particular item offered in the tender or a different make is being proposed by him, the same should be highlighted/mentioned especially by him.

- The prices should	include everything (packing, forwarding, transit insurance, transportation, loading /unloading, installation etc.). Loading/unloading up to 50 mtr. shall be done by the supplier.

- Bid should be submitted for F.O.R delivery at the destination i.e. The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur- 302031

- The bid should be valid for a minimum period of 30 days. The Change in prices shall not be accepted within the validity period.

- The LNMIIT reserves the right to reject any quotation(s) without assigning any reasons.

- Payment terms shall be decided mutually.

- There should not be any conflict of interest (Declaration to be submitted by the Vendor).

- Bid should be on the company’s letterhead and should submit their bid in the LNMIIT format only.

- Item/Service not supplied by the supplier/service provider should be mentioned separately.

- Vendor should provide a photocopy of the cheque/cancelled cheque while presenting the Invoice to LNMIIT.

- Pre-bid meeting and pre-bid site visit can be done before quoting the rate in the tender to clear up any confusion regarding project details, the scope of work, and solicitation of documents.

- All charges applicable shall be clearly mentioned in the offer. If nothing is mentioned in the offer, the quoted price shall be treated as final, and nothing extra shall be paid extra.

All charges applicable shall be clearly mentioned with the offer. If nothing is mentioned in the offer, the quoted price shall be treated as final, and nothing extra shall be paid extra.

Signature with seal