Tender No:. 2024-25/CWO/2B/LNM-819

The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non-Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposals from competent vendors for “Renovation work in the shopping area including the Electrical and Civil Work”. Bidders can submit their sealed offers at the following address by Email/Speed-Post/ Registered Post/ By-hand/courier before 05:00 PM on 13.05.2025 (Last date of bid submission).

Filing of Tender:

Option 1: Physical submission of offers (hard copy) in sealed envelopes.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the institute in person/ Speed-Post/ Registered Post/ By-hand/courier of the Institute after marking due entries in the Receipt Book available with the Security Officer at the main gate.

The tender subject and reference number must be mentioned on the envelope.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Email: directoroffice@lnmiit.ac.in

Option 2: Password-protected online submission via Email

Password-protected offers can be submitted by the bidders via email at the below-mentioned email IDs.

tender-document@lnmiit.ac.in (password-protected document)

tender-password@lnmiit.ac.in (password only)

The tender subject and reference number must be mentioned in the subject line of the emails.

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory, are required to be submitted in two separate envelopes/emails.

Envelopes/Emails should be duly marked as follows:

- Technical bid for the for “Renovation work in the shopping area including the Electrical and Civil Work”.

- Commercial Bid for the for “Renovation work in the shopping area including the Electrical and Civil Work”.

Both Envelopes/files should be enclosed/attached in a single envelope/email bearing the heading for “Renovation work in the shopping area including the Electrical and Civil Work” Tender No.: 2024-25/CWO/2B/LNM-819).

Bids are liable to be rejected:

- If the above-mentioned bid submission procedure is not followed.

- Incomplete and non-conforming bids.

- Arbitration: All disputes and differences arising between the two parties in connection with this order shall be mutually settled as far as possible, failing which, all such disputes shall be referred to arbitration and settlement under the provision of the Arbitration and Conciliation Act, 1996 and as amended from time to time or any such law for the time being prevailing in India. The venue of such arbitration shall be Jaipur.

Technical Bid: (in a separate sealed envelope/via email)

Renovation work in the shopping area including the Electrical and Civil Work” Tender No.: 2024-25/CWO/2B/LNM-819)

While submitting quotations, the following procedure/ formalities and conditions may kindly be adhered.

- Company profile or brief note about the bidder's company, PAN number, and GST number should be submitted by the Vendor.

- The List of Directors/Partners/Proprietor/Members of the Firm to be attached.

- Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members to be provided.

- Turnover for the last three years to be mentioned. (Copy to be enclosed)

- List of the client to whom the vendor has delivered the goods or provided the services in the past.

- The Client List with some purchase orders and satisfactory certificate (to whom the vendor has delivered the goods/provided the services in the past).

- Past performance: Provide list of buyers along with satisfactory reports for installations/ supply of commercial Kitchen equipment in last 03 years. The seller / bidder must have supplied/ installed the commercial usage same/ similar Kitchen equipment to/ at the Government or civil agencies/ Corporations/ Educational Institutes. Preference will be given for above 75% satisfactory report for installation/ supply of commercial Kitchen equipment in last 03 years.

- Completion certificate (attested copy/ true Copy) in respect of supply and installation of the Kitchen equipment worth of at least 20 lakhs at any of the central or state government/ PSU/ PSE/ Educational Institute buyer needs to be attached.

Scope of work, technical data sheet, and Specification of the items:

## Scope of work

- The scope of work will cover the supply and installation of for “Renovation work in the shopping area including the Electrical and Civil Work” at the LNMIIT Jaipur campus.

- The Vendor is required to submit the complete documentation of the supplied item after delivery.

Specification for Items:

  - Supply and installation of electrical wiring for food court common bathrooms

| S.No. | Item Description | UOM | Qty |
| --- | --- | --- | --- |
| 1 | Wiring with 2x1.5sq.mm.(P+N)+.75Sqmm(E) FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires in 25mm dia heavy class recessed system including the cost of cutting / making good chases in brick work FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires complete as per specifications and as required. Make Polycab | Rmtr | 60 |
| 2 | Wiring with 2x4 sq.mm.(P+N)+2.5Sqmm(E) FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires in 25mm dia heavy class recessed and / or surface uPVC electrical conduiting system including the cost of cutting / making good chases in brick work FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires complete as per specifications and as required. Make Polycab | Rmtr | 100 |
| 3 | Supply & Installation of Switch, Socket & Board Make Legrand Mylinc |  |  |
| a | 6A Switch | Nos | 12 |
| b | 6/16 Socket | Nos | 4 |
| c | 16A Switch | Nos | 2 |
| d | 12 Module Box, cover plate with all acessories | Nos | 2 |
| e | 2 Module Box, cover plate with all acessories | Nos | 2 |
| f | Blanker | Nos | 6 |
| 4 | Supply & Installation of 6 Module weather proof cabinate,with mcb's Make Legrand/Hager/Havells/schnider | Nos | 1 |
| a | 16A C type DP | Nos | 3 |
| 5 | Supply & Installation of Led Light Make Phillips |  |  |
| a | Phillips Full Glow LED Surface light,15 Watt | Nos | 8 |

2. Renovation work in the shopping area toilets

| S.No. | Item Description | UOM | Qty |
| --- | --- | --- | --- |
| Part I | DISMANTLING WORK | Unit | Quantity |
| 1.1 | Dismantling tile work in floors and walls laid in cement mortar including stacking of serviceable material and disposal of unserviceable material within 50 meter lead: |  |  |
| 1.1.1 | For thickness of tiles 25mm to 40mm. | SQM | 200.00 |
| 1.2 | Dismantling doors, windows and clearstory windows steel or wood shutter including chowkhats and holdfasts etc. complete and stacking within 50 meters lead: |  |  |
| 1.2.1 | Of area 3 square meter and below. | Each | 6.00 |
| 1.3 | Dismantling of flushing cistern, urinals of any size including stacking of useful materials near the site and disposal of unserviceable material within 50 meter lead. Urinals & their partions will unitized back with urinal jibbis |  |  |
| 1.3.1 | For thickness of tiles 25mm to 40mm. | Each | 8.00 |
| 1.4 | Dismantling of cement concrete platform along with curtain walls, washbasins, base concrete etc. including stacking of useful materials near the site and disposal of unserviceable materials within 50 metres lead |  |  |
| 1.4.1 | Above 2.52 Sqm | SQM | 8.00 |
| Part II | REINFORCED CEMENT CONCRETE WORK |  |  |
| 2.1 | Providing, laying & testing cast - in - place structural controlled cement concrete grade M25 using 20mm down coarse aggregates but including cost of centering & cement, shuttering and steel reinforcement but including cost of making shear keys and provision of construction / contraction / control joints in various locations, laid, consolidated and cured. Item to include all structural items like base raft, ramp, column, wall, footings, equipment foundations, fins, mullions, beams, lintels, pergola, chajja, facia, parapet, railing, slab trenches, drains and sumps etc. The rates of concrete to include the cost of admixtures / plasticizers where required . This item to also include the concrete requiring waterproofing like water tanks, retaining walls, base raft, etc. |  |  |
| 2.1.1 | counters | CUM | 0.80 |
| Part III | BRICK WORK |  |  |
| 3.1 | Half brick masonry using selected quality burnt clay FPS brick of class designation 75 [Crushing strength not less than 75 kg/sqcm] in superstructure laid in cement mortar 1:4 (1 cement: 4 coarse sand) mix, joints finished, flush raked to 6mm depth as directed by Project Manager including providing & placing 2 nos. 8mm dia MS bars reinforcement at every fourth course, curing, anchoring with concrete as per specification and drawing or as directed by Estate Manager. |  |  |
| 3.1.1 | Under the counter top | SQM | 6.00 |
| Part IV | WOOD WORK |  |  |
| 4.1 | Providing and fixing pressed steel door frames manufactured from commercial mild steel sheet of 1.25 mm thickness including hinges jamb, lock jamb, bead and if required angle threshold of mild steel angle of section 50 x 25mm, or base ties of 1.25mm pressed mild steel welded or rigidly fixed together by mechanical means, adjustable lugs with mortar guards, lock strike-plate and shock absorbers as specified and applying a coat of approved steel primer after pre-treatment of the surface as directed. |  |  |
| 4.1.1 | Profile A | RMT | 33.00 |
| 4.2 | Providing and fixing 35 mm ( overall) thick both side commercial type flush door shutters , core of block board construction and frame of 1st class hardwood phenol bonded, 12mm thick wood lipping on all edges of shutters including 4 nos. brushed steel heavy duty butt hinges 125 mm with matching screws , PVC buffers, polishing where required etc. all complete as per design and drawing. ( The thickness of shutter in all the cases should not be less than 35mm either veneered or laminated finish) (Cost of providing door seal to be included in the quoted rate). | SQM | 12.60 |
| 4.3 | Providing and fixing tower bolt (barrel type) with necessary screws etc. complete. |  |  |
| 4.3.1 | 250 mm x 10 mm SS | EACH | 6.00 |
| 4.4 | Providing and fixing aluminium handles with necessary screws etc. complete. |  |  |
| 4.4.4 | 150 mm | EACH | 12.00 |
| 4.5 | Providing and fixing sliding door bolts/ latch |  |  |
| 4.5.1 | 300 mm | EACH | 6.00 |
| 4.6 | Front Door stopper | EACH | 2.00 |
| 4.7 | Back Door stopper | EACH | 2.00 |
| 4.8 | Door Closer of godrej | EACH | 2.00 |
| Part V | FLOOR AND WALL TILES |  |  |
| 5.1 | Providing and laying tile flooring of approval make, shade as approved by Architect over adequate thickness of mortar (1:4), making joint with PVC spacers of 3 mm to be filled with tile grout with approved pigmented to match the shade of tile & laid as per design & drawing approved by Architect for various thick of tiles but inculding cost of cement. |  |  |
| 5.1.1 | Ceramic tile ( basic rate Rs. 50 per sqft ) (200 x 300 mm) | SQM | 150.00 |
| 5.1.2 | Vitrified tile (basic rate Rs. 80 per sqft) (400 x 400mm) | SQM | 70.00 |
| 5.2 | Providing and fixing prepolished 20mm thick stone slab for the toilet counter & facial laid over 12mm thick cement mortar 1:4 (1 cement : 4 coarse sand) inclusive of making and rounding off edges and cutting holes (circular or oval) for wash basin, MS angle to fix bottom stone including rubbing polishing complete including cost of cement. (Basic rate of granite as per SCC - 250 / sq ft) |  |  |
| 5.2.1 | Counter top/ Facial Dado granite | SQM | 10.00 |
| Part VI | PLUMBING FIXTURES |  |  |
| 6.1 | Providing & fixing vitreous china oval (Table top) wash basin 550x400mm dia. for under counter mounting, specially fabricated brackets painted white, 32mm C.P.cast brass bottle trap and pipe to wall with C.P. wall flange and rubber adapter for waste connection complete, including cutting and making good the walls wherever required. |  |  |
| 6.1.1 | White colour (Cera Catalog No - 1087) | EACH | 6.00 |
| 6.2 | Providing and fixing white vitreous China Indian water closet with 100mm sand cast iron ‘P‘ or ‘S‘ trap set in cement concrete 1:2:4 mix (1 cement :2 coarse sand :4 graded stone aggregate 20mm nominal size) with 10 litre capacity high or low level PVC cistern with top button or front handle operation along with 32mm dia PVC flush pipe with clamps and rubber adapter joint with C.P. brass screws and washer complete, including cutting and making good the walls and floors etc. Indian type Orissa Pattern size 580x440mm. |  |  |
| 6.2.1 | White colour ( 2071- W.C. & 2604- Cistern) | EACH | 4.00 |
| 6.3 | Providing & fixing 15mm dia C.P. brass bib tap complete including cutting and making good the walls wherever required. |  |  |
| 6.3.1 | Two way Bib Cock (ESS ESS, Croma series, code- C-203) | EACH | 4.00 |
| 6.4 | Providing & fixing 15mm C.P brass angle valve with C.P. copper connecting pipe 375mm long & nuts, washer and C.P. brass flange complete, including cutting & making good the wall wherever required. ((ESS ESS, Croma series, code- C-204) | EACH | 10.00 |
| 6.5 | Providing & fixing 15mm C.P. pillar tap for W.B.(ESS ESS, Croma series, code- C-201) | EACH | 6.00 |
| 6.6 | Providing and fixing Chlorinated polyvinyl Chloride pipes (CPVC) (ASTM F437 & ASTM F438) SDR 13.5 having thermal stability for hot & cold water supply system including all CPVC fitting like plain & brass threaded fittings including cutting and chasing in walls and floors and repairing these in good conditions. This includes jointing pipe & fittings one step with the CPVC solvent cement & testing of joints complete as per direction of the Engineer in-charge |  |  |
| 6.6.1 | 15 MM | EACH | 6.00 |
| 6.7 | Bottle Trap PVC (Prayag) | EACH | 8.00 |
| 6.8 | Washbasin/ Urinal Waste Coupling (Prayag) | EACH | 8.00 |
| 6.9 | PVC Connection Pipe 18 inch (Asian Paint ESS ESS B2CH218) | EACH | 10.00 |
| PartVII | PAINTING |  |  |
| 7.1 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade including all scaffolding: Old work (One or more coats)on new work including preparation of base with primer, putty, lippy etc. complete in all respect.Application : Director House and B Qtr. | SQM | 90.00 |
| 7.2 | Painting with synthetic enamel paint of approved brand and manufacture including scrubbing through sandpaper, and red oxide, to give an even shade: One or more coats on old work. Application: MS Windows (Windows Grills), Wooden Door, Frames and wall of hostel/ACA | SQM | 65.00 |

Financial Bid: (in a separate sealed envelope/password protected file via email)

Renovation work in the shopping area including the Electrical and Civil Work” Tender No.: 2024-25/CWO/2B/LNM-819)

| S.No. | Item Description | UOM | Qty | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Wiring with 2x1.5sq.mm.(P+N)+.75Sqmm(E) FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires in 25mm dia heavy class recessed system including the cost of cutting / making good chases in brick work FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires complete as per specifications and as required. Make Polycab | Rmtr | 60 |  |  |
| 2 | Wiring with 2x4 sq.mm.(P+N)+2.5Sqmm(E) FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires in 25mm dia heavy class recessed and / or surface uPVC electrical conduiting system including the cost of cutting / making good chases in brick work FRLS PVC insulated 660/1100 volt grade stranded copper conductor wires complete as per specifications and as required. Make Polycab | Rmtr | 100 |  |  |
| 3 | Supply & Installation of Switch, Socket & Board Make Legrand Mylinc |  |  |  |  |
| a | 6A Switch | Nos | 12 |  |  |
| b | 6/16 Socket | Nos | 4 |  |  |
| c | 16A Switch | Nos | 2 |  |  |
| d | 12 Module Box, cover plate with all acessories | Nos | 2 |  |  |
| e | 2 Module Box, cover plate with all acessories | Nos | 2 |  |  |
| f | Blanker | Nos | 6 |  |  |
| 4 | Supply & Installation of 6 Module weather proof cabinate,with mcb's Make Legrand/Hager/Havells/schnider | Nos | 1 |  |  |
| a | 16A C type DP | Nos | 3 |  |  |
| 5 | Supply & Installation of Led Light Make Phillips |  |  |  |  |
| a | Phillips Full Glow LED Surface light,15 Watt | Nos | 8 |  |  |
|  |  |  | Electrical work | Total (A) |  |

2. Renovation work in the shopping area toilets

| S.No. | Item Description | UOM | Qty | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| Part I | DISMANTLING WORK |  |  |  |  |
| 1.1 | Dismantling tile work in floors and walls laid in cement mortar including stacking of serviceable material and disposal of unserviceable material within 50 meter lead: |  |  |  |  |
| 1.1.1 | For thickness of tiles 25mm to 40mm. | SQM | 200.00 |  |  |
| 1.2 | Dismantling doors, windows and clearstory windows steel or wood shutter including chowkhats and holdfasts etc. complete and stacking within 50 meters lead: |  |  |  |  |
| 1.2.1 | Of area 3 square meter and below. | Each | 6.00 |  |  |
| 1.3 | Dismantling of flushing cistern, urinals of any size including stacking of useful materials near the site and disposal of unserviceable material within 50 meter lead. Urinals & their partions will unitized back with urinal jibbis |  |  |  |  |
| 1.3.1 | For thickness of tiles 25mm to 40mm. | Each | 8.00 |  |  |
| 1.4 | Dismantling of cement concrete platform along with curtain walls, washbasins, base concrete etc. including stacking of useful materials near the site and disposal of unserviceable materials within 50 metres lead |  |  |  |  |
| 1.4.1 | Above 2.52 Sqm | SQM | 8.00 |  |  |
| Part II | REINFORCED CEMENT CONCRETE WORK |  |  |  |  |
| 2.1 | Providing, laying & testing cast - in - place structural controlled cement concrete grade M25 using 20mm down coarse aggregates but including cost of centering & cement, shuttering and steel reinforcement but including cost of making shear keys and provision of construction / contraction / control joints in various locations, laid, consolidated and cured. Item to include all structural items like base raft, ramp, column, wall, footings, equipment foundations, fins, mullions, beams, lintels, pergola, chajja, facia, parapet, railing, slab trenches, drains and sumps etc. The rates of concrete to include the cost of admixtures / plasticizers where required . This item to also include the concrete requiring waterproofing like water tanks, retaining walls, base raft, etc. |  |  |  |  |
| 2.1.1 | counters | CUM | 0.80 |  |  |
| Part III | BRICK WORK |  |  |  |  |
| 3.1 | Half brick masonry using selected quality burnt clay FPS brick of class designation 75 [Crushing strength not less than 75 kg/sqcm] in superstructure laid in cement mortar 1:4 (1 cement: 4 coarse sand) mix, joints finished, flush raked to 6mm depth as directed by Project Manager including providing & placing 2 nos. 8mm dia MS bars reinforcement at every fourth course, curing, anchoring with concrete as per specification and drawing or as directed by Estate Manager. |  |  |  |  |
| 3.1.1 | Under the counter top | SQM | 6.00 |  |  |
| Part IV | WOOD WORK |  |  |  |  |
| 4.1 | Providing and fixing pressed steel door frames manufactured from commercial mild steel sheet of 1.25 mm thickness including hinges jamb, lock jamb, bead and if required angle threshold of mild steel angle of section 50 x 25mm, or base ties of 1.25mm pressed mild steel welded or rigidly fixed together by mechanical means, adjustable lugs with mortar guards, lock strike-plate and shock absorbers as specified and applying a coat of approved steel primer after pre-treatment of the surface as directed. |  |  |  |  |
| 4.1.1 | Profile A | RMT | 33.00 |  |  |
| 4.2 | Providing and fixing 35 mm ( overall) thick both side commercial type flush door shutters , core of block board construction and frame of 1st class hardwood phenol bonded, 12mm thick wood lipping on all edges of shutters including 4 nos. brushed steel heavy duty butt hinges 125 mm with matching screws , PVC buffers, polishing where required etc. all complete as per design and drawing. ( The thickness of shutter in all the cases should not be less than 35mm either veneered or laminated finish) (Cost of providing door seal to be included in the quoted rate). | SQM | 12.60 |  |  |
| 4.3 | Providing and fixing tower bolt (barrel type) with necessary screws etc. complete. |  |  |  |  |
| 4.3.1 | 250 mm x 10 mm SS | EACH | 6.00 |  |  |
| 4.4 | Providing and fixing aluminium handles with necessary screws etc. complete. |  |  |  |  |
| 4.4.4 | 150 mm | EACH | 12.00 |  |  |
| 4.5 | Providing and fixing sliding door bolts/ latch |  |  |  |  |
| 4.5.1 | 300 mm | EACH | 6.00 |  |  |
| 4.6 | Front Door stopper | EACH | 2.00 |  |  |
| 4.7 | Back Door stopper | EACH | 2.00 |  |  |
| 4.8 | Door Closer of godrej | EACH | 2.00 |  |  |
| Part V | FLOOR AND WALL TILES |  |  |  |  |
| 5.1 | Providing and laying tile flooring of approval make, shade as approved by Architect over adequate thickness of mortar (1:4), making joint with PVC spacers of 3 mm to be filled with tile grout with approved pigmented to match the shade of tile & laid as per design & drawing approved by Architect for various thick of tiles but inculding cost of cement. |  |  |  |  |
| 5.1.1 | Ceramic tile ( basic rate Rs. 50 per sqft ) (200 x 300 mm) | SQM | 150.00 |  |  |
| 5.1.2 | Vitrified tile (basic rate Rs. 80 per sqft) (400 x 400mm) | SQM | 70.00 |  |  |
| 5.2 | Providing and fixing prepolished 20mm thick stone slab for the toilet counter & facial laid over 12mm thick cement mortar 1:4 (1 cement : 4 coarse sand) inclusive of making and rounding off edges and cutting holes (circular or oval) for wash basin, MS angle to fix bottom stone including rubbing polishing complete including cost of cement. (Basic rate of granite as per SCC - 250 / sq ft) |  |  |  |  |
| 5.2.1 | Counter top/ Facial Dado granite | SQM | 10.00 |  |  |
| Part VI | PLUMBING FIXTURES |  |  |  |  |
| 6.1 | Providing & fixing vitreous china oval (Table top) wash basin 550x400mm dia. for under counter mounting, specially fabricated brackets painted white, 32mm C.P.cast brass bottle trap and pipe to wall with C.P. wall flange and rubber adapter for waste connection complete, including cutting and making good the walls wherever required. |  |  |  |  |
| 6.1.1 | White colour (Cera Catalog No - 1087) | EACH | 6.00 |  |  |
| 6.2 | Providing and fixing white vitreous China Indian water closet with 100mm sand cast iron ‘P‘ or ‘S‘ trap set in cement concrete 1:2:4 mix (1 cement :2 coarse sand :4 graded stone aggregate 20mm nominal size) with 10 litre capacity high or low level PVC cistern with top button or front handle operation along with 32mm dia PVC flush pipe with clamps and rubber adapter joint with C.P. brass screws and washer complete, including cutting and making good the walls and floors etc. Indian type Orissa Pattern size 580x440mm. |  |  |  |  |
| 6.2.1 | White colour ( 2071- W.C. & 2604- Cistern) | EACH | 4.00 |  |  |
| 6.3 | Providing & fixing 15mm dia C.P. brass bib tap complete including cutting and making good the walls wherever required. |  |  |  |  |
| 6.3.1 | Two way Bib Cock (ESS ESS, Croma series, code- C-203) | EACH | 4.00 |  |  |
| 6.4 | Providing & fixing 15mm C.P brass angle valve with C.P. copper connecting pipe 375mm long & nuts, washer and C.P. brass flange complete, including cutting & making good the wall wherever required. ((ESS ESS, Croma series, code- C-204) | EACH | 10.00 |  |  |
| 6.5 | Providing & fixing 15mm C.P. pillar tap for W.B.(ESS ESS, Croma series, code- C-201) | EACH | 6.00 |  |  |
| 6.6 | Providing and fixing Chlorinated polyvinyl Chloride pipes (CPVC) (ASTM F437 & ASTM F438) SDR 13.5 having thermal stability for hot & cold water supply system including all CPVC fitting like plain & brass threaded fittings including cutting and chasing in walls and floors and repairing these in good conditions. This includes jointing pipe & fittings one step with the CPVC solvent cement & testing of joints complete as per direction of the Engineer in-charge |  |  |  |  |
| 6.6.1 | 15 MM | EACH | 6.00 |  |  |
| 6.7 | Bottle Trap PVC (Prayag) | EACH | 8.00 |  |  |
| 6.8 | Washbasin/ Urinal Waste Coupling (Prayag) | EACH | 8.00 |  |  |
| 6.9 | PVC Connection Pipe 18 inch (Asian Paint ESS ESS B2CH218) | EACH | 10.00 |  |  |
| PartVII | PAINTING |  |  |  |  |
| 7.1 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade including all scaffolding: Old work (One or more coats)on new work including preparation of base with primer, putty, lippy etc. complete in all respect.Application : Director House and B Qtr. | SQM | 90.00 |  |  |
| 7.2 | Painting with synthetic enamel paint of approved brand and manufacture including scrubbing through sandpaper, and red oxide, to give an even shade: One or more coats on old work. Application: MS Windows (Windows Grills), Wooden Door, Frames and wall of hostel/ACA | SQM | 65.00 |  |  |
|  |  |  | Civil Work | Total ((B) |  |
|  |  |  | Civil work + electrical work | Total (A+B) |  |
|  |  |  |  | GST |  |
|  |  |  |  | Total with GST |  |

Other T&C

- Taxes like GST should be clearly mentioned. If nothing is mentioned, these charges shall be deemed included.

- If the supplier does not have a particular item offered in the tender or a different make is being proposed by him, the same should be highlighted/mentioned especially by him.

- The prices should	include everything (packing, forwarding, transit insurance, transportation, loading /unloading, installation etc.). Loading/unloading up to 50 mtr. shall be done by the supplier.

- Bid should be submitted for F.O.R delivery at the destination i.e. The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur- 302031

- The bid should be valid for a minimum period of 30 days. The Change in prices shall not be accepted within the validity period.

- The LNMIIT reserves the right to reject any quotation(s) without assigning any reasons.

- Payment terms shall be decided mutually.

- There should not be any conflict of interest (Declaration to be submitted by the Vendor).

- Bid should be on the company’s letterhead and should submit their bid in the LNMIIT format only.

- Item/Service not supplied by the supplier/service provider should be mentioned separately.

- Vendor should provide a photocopy of the cheque/cancelled cheque while presenting the Invoice to LNMIIT.

- Pre-bid meeting and pre-bid site visit can be done before quoting the rate in the tender to clear up any confusion regarding project details, the scope of work, and solicitation of documents.

- All charges applicable should be clearly mentioned in the offer. If nothing is mentioned in the offer, the quoted price shall be treated as final, and nothing extra shall be paid extra.

Signature with seal