TECHNICAL SPECIFICATION FOR ROAD INFRA WORKS

- This pertains to entire road work and paving outside the building battery limits (it may be noted that paving within the building battery limit is part of building scope – however all the work falls part of contractor’s scope);

- Scope shall include the entire development of roads & pathways from excavation to hand over including laying of MEPF services from within the road, services crossing, etc. augmentation with any existing road, etc. complete.

- Scope includes all roads, kerbs, surface drains & pathways (incl. walkway) within the project site as defined in the layout / drawings.

- Site clearance of existing Malba and waste of site clearance to be carted away, outside project boundary at the contractor’s arrangements and place of dumping – the said site of dumping shall have to be organised by contractor without any liability whatsoever and/or extra cost to LNMIT.

- Road work shall be for granular subbase of 150mm thick, WMM (Wet Mix Macadam) based road of 150mm thk.( Compacted thickness ) finished with Cement concrete pavement including mixing and vibrations concrete with necessary needle vibrators and surface with screed board vibrating using vacuum dewatering process complete in all respect including anti-skid textured finish to required camber. The ground shall be excavated for the laying of the entire base, subbase, etc. and shall be systematically layered, compacted, laid as per best engineering practice for the said work;

- The scope of earthwork shall be inclusive of surface dressing, excavation, filling, disposal of surplus / unwanted soil within the 2.0 km lead boundary of LNMIT campus, as per direction of LNMIT-EIC. All waste of site clearance to be carted away, outside the project site at the contractor’s arrangements and place of dumping – the said site of dumping shall have to be organised by contractor without any liability whatsoever and/or extra cost to LNMIT.

- The sequence of laying layers for the road shall be:

- Surface preparation

- Granular subbase

- WMM

- Cement concrete pavement (M-30 Grade)

- The depth for excavation shall be as per the thickness of the subgrade mentioned above. The excavated surface is then compacted with the 8-10 tonne vibro-roller.

- The compaction shall be carried along with any watering, filling required to give even finished top surface. The vibro roller shall be compact the surfaces from side edges to centre with parallel movements to edge of the road.

- The excavated part of land shall be compacted and made rough to received the layer of GSB layer and final layer of WMM compacted thickness of 150mm.

- The surface shall be broomed to make it rough with ponding method of curing.

- Granular subbase shall be mixed in a mechanical mix plant at OMC and spreading it in uniform layers of specified thickness with motor grader on prepared surface and compacting with vibratory power roller to achieve the desired density.

- The Wet Mix Macadam with Stone Aggregate: Stone aggregate of size as per Indian Road Congress Journal (IRC) latest revisions.

- The coarse aggregate shall be spread uniformly and evenly upon the prepared sub-grade in required quantities with a twisting motion to avoid segregation.

- Dumping of the aggregates in the heaps shall be strictly avoided.

- The WMM layer shall be compacted to desire thickness of minimum 150 mm (compacted) thickness.

- The M-30 grade cement concrete pavement should be processed with vacuum dewatering system, including of surface vibrating and levelling the surface  with screed vibrator, vacuum dewatering the surface, power trowelling the surface to make it smooth and  finished, including groove cutting upto 20 mm deep and 5 mm wide construction/Expansion joints and making a  panel of approx 3.0 mt. x 4.0 mt  approx or as approved by LNMIT-EIC. These joints are to be filled with polysulphide sealant as approved and directed.

- The road shall have camber along the centre line of the road. The slope is provided towards the edges of the road for easy disposal of rain/storm water.

- The parking area shall be finished with the 75 mm thick factory-made cement concrete precast RCC grass grid pavers of Grade M-30. made by block making machine with strong vibratory compaction of approved size, design and shape over the 50mm thick compacted layer of compacted bed of course sand.

- The factory-made kerb stone shall be fixed in PCC laid in the trenches made. The layer of WMM to be extended outside by minimum 150 mm, on which 100 mm thick PCC of 1:4:8 will be done, for kerb stone fixing. Offset in road for Storm Water Chamber area to have PCC (1:4:8) of 150 mm thick in slope towards the chamber. Kerb stone will also divert from straight line and will be laid along this offset for the area of the storm chamber; The kerb stone shall be painted (2 coats of primer, 2-3 coats of enamel paint of dual shade – yellow & black)at no extra cost;

- The top level of manhole cover and top surface of the road shall be at the same level.

- The road shall have speed breakers of make ‘Dark Eye’ or equivalent make to be placed at locations marked in drawing.

- Scope shall include, longitudinal drain in PCC (1:2:4) of 300 mm width and avg. 100 mm thickness to be provided along the entire length (both sides) of the roads with slope towards storm water chamber;

- The scope shall also include the maintaining roads, replacement/ repairs of broken, inferior quality of pavers trough out the project period and till the end of DLP;

- Rectification of snags / defects shall needs to be done by contractor within a week's time frame, in case he fails to do so, he is liable to penalty as per scale & discretion of EIC; however in extreme case, where contractor is non-responsive, LNMIT shall get the same work / rectification done from another agency at the risk & cost of the contractor;

- All the above listing is done so as to provide a brief summary about the road development, the same may not be exhaustive and shall need to be read and understood along with issued drawings, technical specifications, etc.;