Tender No:. 2025-26/Estate/2B/LNM-1217

The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non-Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposals from competent vendors for “Renovation of Guest House and ACA Faculty gate Toilet Block”. Bidders can submit their sealed offers/s at the following address by Email/Speed-Post/ Registered Post/ By-hand/courier before 05:00 PM on 24/12/2025.

Filing of Tender:

Option 1: Physical submission of offers (hard copy) in sealed envelopes.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the institute in person/ Speed-Post/ Registered Post/ By-hand/courier of the Institute after marking due entries in the Receipt Book available with the Security Officer at the main gate.

The tender subject and reference number must be mentioned on the envelope.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Option 2: Password-protected online submission via Email

Password-protected offers can be submitted by the bidders via email at the below-mentioned email IDs.

tender-document@lnmiit.ac.in (password-protected document)

tender-password@lnmiit.ac.in (password only)

The tender subject and reference number must be mentioned in the subject line of the emails.

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory, are required to be submitted in two separate envelopes/emails.

Envelopes/Emails should be duly marked as follows:

- Technical bid for the “Renovation of Guest House and ACA Faculty gate Toilet Block ”.  (Technical bid shall be without password protected)

- Commercial Bid for the “Renovation of Guest House and ACA Faculty gate Toilet Block ”. (Commercial bid shall be password protected)

Both Envelopes/files should be enclosed/attached in a single envelope/email bearing the heading “Bid for the “Renovation of Guest House and ACA Faculty gate Toilet Block ”) (Tender No. : 2025-26/Estate/2B/LNM-1217) ”.

Note: The bidders/vendors are requested to go through the tender document carefully and ensure compliance with all specifications/instructions in this tender

Bids are liable to be rejected:

- If the above-mentioned bid submission procedure is not followed.

- Incomplete and non-conforming bids.

- Arbitration: All disputes and differences arising between the two parties in connection with this order shall be mutually settled as far as possible, failing which, all such disputes shall be referred to arbitration and settlement under the provision of the Arbitration and Conciliation Act, 1996 and as amended from time to time or any such law for the time being prevailing in India. The venue of such arbitration shall be Jaipur.

Technical Bid: (in a separate sealed envelope/via email)

# Bid for the Renovation of Guest House and ACA Faculty gate Toilet Block

# (Tender No. : 2025-26/Estate/2B/LNM-1217)

Bidder is required to furnish the requisite details as per the below mentioned table format and proof of such documents to be uploaded along with the bid under respective title/ headings as mentioned at column no ‘C’ accordingly.

| S.No. | Title/ Heading Details sought by buyer | Details/ Certificate No. (Attached Y/N) |
| --- | --- | --- |
| A | B | C |
| 1. | Bidder Firm Name & Registration No, |  |
| 2. | Bidder’s Company profile or brief note |  |
| 3. | PAN Card No,. |  |
| 4. | GST Number |  |
| 5. | List of Directors/ Partners/Proprietor/ Members of the Bidder’s Firm |  |
| 6. | Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members |  |
| 7. | Bidder Turnover - Average annual turnover for last three years |  |
| 8. | The Client List with some purchase orders and satisfactory certificate (to whom the vendor has delivered the goods/provided the services in the past (Three (03) purchase orders/work orders, at least one from each of the last three years, including the previous year.) |  |
| 9. | If the bidder is authorized by the OEM/Manufacturer. Proof of the same is to be submitted with the Technical Bid |  |
| 10. | An undertaking (self-certificate) that the agency has not been blacklisted by a Central/State/UT Government Institutions and there has been no litigation with any government department/institution on account of any services/supplies. |  |
| 11. | Scanned copy of EMD/ Tender Fees paid through RTGS/NEFT (if required/asked in the tender) |  |
| 12 | Xerox copies of MoUs with previous colleges (minimum three) must be attached. (Applicable for student fest only). |  |

Scope of work

- The scope of work will cover the supply /arrangement of given items at LNMIIT Jaipur campus.

- The Vendor is required to submit the complete documentation of the supplied item after delivery.

1. Scope of Work – Civil, Plumbing, Sanitary, Electrical & Enabling Works

This scope of work covers complete renovation of existing toilet blocks, including dismantling, civil repairs, flooring, doors, plumbing, sanitary fixtures, electrical works, waterproofing, testing and commissioning, and all enabling works required for safe and uninterrupted execution.

2. Enabling Works (Pre-Construction)

The contractor shall carry out all enabling works necessary for smooth execution of the project, including but not limited to:

- Site preparation and safety

  - Barricading of work areas with caution signage.

  - Provision of temporary lighting, safety boards, PPE for workers.

  - Protection of adjacent rooms, corridors, shafts, and existing services.

- Temporary arrangements

- Temporary diversion/protection of existing water supply, drainage and electrical lines to avoid disruption.

- Temporary storage space for dismantled serviceable materials.

- Temporary arrangement for debris collection and disposal.

- Survey & setting out

  - Detailed site measurement, verification of existing pipe sizes, slopes, electrical points, fixtures, and levels.

  - Marking of new pipeline routes, fixture locations, and electrical points before execution.

- Debris handling

  - Segregation of serviceable and unserviceable materials.

  - Stacking of serviceable materials within 50 m lead.

  - Disposal of debris to approved dumping locations as per local authority norms.

3. Dismantling & Demolition Works

- Dismantling of existing floor and wall tiles including removal of mirrors, glasses, fixtures, stacking of reusable materials and disposal of debris within 50 m lead.

- Dismantling of doors, windows, ventilators, chowkhats, holdfasts and frames (steel/wood), including stacking and disposal.

- Removal of existing WC/EWC, urinals, cisterns, partitions and sanitary accessories, including safe disconnection from plumbing lines.

- Dismantling of RCC platforms, counters, curtain walls, wash basin bases and associated concrete works.

- Dismantling of existing sewage and water supply pipelines (PVC/CI/GI/CPVC/UPVC), valves, fittings and supports.

4. Plumbing & Sanitary Works

- Drainage system

  - Providing and laying new sewage drainage pipes (HDPE/PVC – 40/75/110 mm or existing sizes) including traps, bends, sockets, elbows, and necessary fittings.

  - Trenching, bedding, PCC below pipes, jointing, testing, backfilling and making good the surfaces.

- Water supply system

  - Providing and laying new CPVC/UPVC water supply lines (20/25/32 mm or existing sizes) including tees, elbows, sockets, valves and supports.

  - Cutting and chasing in walls/floors and restoring surfaces.

  - Testing and flushing of complete water supply system.

- Sanitary fixtures

  - Supply and installation of wash basins, Indian WC/EWC, urinals with partitions, cisterns, traps, bottle traps, waste couplings, health faucets, bib taps, pillar taps, angle valves and other accessories as specified.

  - Proper alignment, sealing, testing for leakage and making good adjacent finishes.

5. Civil & Finishing Works

- PCC works below pipelines and for bedding, foundations and repair works as specified.

- Structural concrete works (M25) for counters, platforms or minor structural repairs, including shuttering and curing.

- Tile flooring and wall dado using approved ceramic/vitrified tiles, laid in cement mortar with spacers, grouting and finishing complete.

- Granite/stone counter tops and facia with edge polishing, cut-outs for wash basins and proper fixing.

- Supply and fixing of mirrors with SS designer bolts and proper alignment.

- Waterproofing treatment on toilet walls including surface preparation, chemical treatment, plaster repair, primer, putty and finishing coats.

6. Doors & Joinery Works

- Providing and fixing stone/metal door frames including primer and finishing paint.

- Providing and fixing flush doors with hardwood lipping, hinges, door seals and all hardware.

- Supply and fixing of tower bolts, aldrops, handles, sliding bolts, door stoppers and door closers as specified.

7. Electrical Works (Complete Toilet Electrical Renovation)

The electrical scope shall include supply, installation, testing and commissioning of all electrical works if required for toilet blocks:

- Dismantling & making safe

  - Removal of existing damaged/obsolete wiring, switches, fittings and fixtures.

  - Making safe of all live electrical points during demolition.

- Wiring & conduits

  - Concealed PVC conduit wiring with FRLS copper cables from nearest distribution board.

  - Separate circuits for lighting, exhaust fans and power points.

  - Proper earthing for all fixtures and equipment.

- Lighting & fixtures

  - Providing and fixing LED light fittings suitable for wet areas.

  - Mirror lights where required.

  - Replacement/installation of bulkhead or surface-mounted fittings as approved.

- Switches & accessories

  - Modular switches, sockets, fan regulators and junction boxes of approved make.

  - Installation of weatherproof switches where required.

- Ventilation

  - Providing and fixing exhaust fans with louvers and necessary wiring.

- Distribution & safety

  - Connection to existing DB including MCBs/RCCBs as required.

  - Proper labeling of circuits and testing of insulation resistance, continuity and earthing.

8. Testing, Commissioning & Completion

- Pressure testing of water supply lines and leakage testing of drainage lines.

- Functional testing of all sanitary fixtures, taps, cisterns and accessories.

- Testing and commissioning of electrical installations.

- Final cleaning of toilet blocks, removal of debris, and handing over in clean and usable condition.

9. General Conditions

- All works shall conform to CPWD/PWD specifications, relevant IS codes and local authority norms.

- Materials shall be of approved make and quality.

- Rates shall be inclusive of all labour, materials, tools, scaffolding, safety measures, leads and lifts.

- Work shall be executed with minimum disturbance to occupants and existing services.

- The Work is to commence immediately on issue of PO and is to be completed within 10 day of PO.

Technical specification:

| Sr. No. | Item Name | QTY. | UOM |
| --- | --- | --- | --- |
| A | Civil Work |  |  |
| 1.00 | Dismantling tile work in floors and walls (including removing of Mirrors, glasses, etc.) laid in cement mortar including stacking of serviceable material and disposal of unserviceable material within 50-meter lead: |  |  |
| 1.01 | For thickness of tiles 25mm to 50mm | 285.57 | sqm |
| 2.00 | Dismantling doors, windows and clearstory windows steel or wood shutter including chowkhats and holdfasts etc. complete and stacking within 50 meters lead: |  |  |
| 2.01 | Of area 3 square meter and below. | 12.00 | Each |
| 3.00 | Dismantling of flushing (WC/EWC) with cistern, urinals of any size including stacking of useful materials near the site and disposal of unserviceable material within 50 meter lead. Urinals & their partions will unitized back with urinal jibbis complete in all respects. |  |  |
| 3.01 | For thickness of tiles 25mm to 50mm. | 13.00 | Each |
| 4.00 | Dismantling of light reinforced cement concrete platform along with curtain walls, washbasins, base concrete etc. including stacking of useful materials near the site and disposal of unserviceable materials within 50 metres lead, complete in all respects. |  |  |
| 4.01 | Above 2.52 Sqm | 5.91 | sqm |
| 5.00 | Dismantling of old sewage drainage pipes in lavatory & partial shaft (say PVC/CI) including removal, cutting, stacking, disposal within lead 50 metres. Complete in all respects. | 52.08 | metre |
| 6.00 | Dismantling of existing water supply lines in lavatory & partial shaft (GI/CPVC/UPVC), including valves/fittings/ facuets,etc from riser/trunk including removal, disposal. Complete in all respects. | 53.14 | metre |
| 7.00 | Providing & laying new sewage drain pipe (e.g. HDPE/PVC- Size 110/75/40 mm) of specified diameter (existing dia, with all necessary fittings - eg. Nani Trap, P-Trap, Sockets, Elbows, etc.) including trenching, bedding, jointing, testing, backfill. | 52.08 | metre |
| 8.00 | Providing & laying new water supply line (e.g. CPVC/UPVC- Size 20/25/ 32 mm) of specified diameter (existing dia, with all necessary fittings- Tee, Sockets, Elbows, etc.) including fittings, testing, flushing. | 53.14 | metre |
| 9.00 | Providing and laying Plain Cement Concrete (PCC) of mix 1:4:8 (M7.5) with 40 mm nominal size stone aggregate in foundation and bedding below pipeline, including compacting with plate vibrator, curing, etc., complete in all respects as per the engineers and CPWD/ PWD Norms . | 31.25 | cum |
| 10.00 | Providing and laying tile flooring of approval make, shade as approved by Architect over adequate thickness of mortar (1:4), making joint with PVC spacers of 3 mm to be filled with tile grout with approved pigmented to match the shade of tile & laid as per design & drawing approved by Architect for various thick of tiles but including cost of cement. |  |  |
| 10.01 | Vitrified tile Somany ( basic rate Rs. 40 per sqft ) (Grande callis brown fp -600 x 1200 mm) | 257.79 | sqm |
| 10.02 | Vitrified tile Somany (basic rate Rs. 38 per sqft) (Delmon beige - 600 x 600mm) | 55.54 | sqm |
| 11.00 | Providing and fixing Bijoliya Stone Door frames including hinges jamb, lock Jab, section 110x45mm, or base ties of rigidly fixed together by mechanical means adjustable lugs with mortar guards, lock strike-plate and shock absorbers as specified and applying a coat of approved steel primer & Paint after pre-treatment of the surface as directed. | 66.00 | metre |
| 12.00 | Providing and fixing 35 mm (overall) thick both side commercial type flush door shutters , core of block board construction and frame of 1st class hardwood phenol bonded, 12mm thick wood lipping on all edges of shutters including 4 nos. brushed steel heavy duty butt hinges 125 mm with matching screws , PVC buffers, polishing where required etc. all complete as per design and drawing. (The thickness of shutter in all the cases should not be less than 35mm either veneered or laminated finish) (Cost of providing door seal to be included in the quoted rate). | 25.20 | sqm |
| 12.01 | Providing and fixing tower bolt (barrel type)/ Aldrop SS with necessary screws etc. complete. 250 mm x 10 mm SS | 12.00 | Each |
| 12.02 | Providing and fixing aluminium handles with necessary screws etc. complete. 150 mm | 12.00 | Each |
| 12.03 | Providing and fixing sliding door bolts/ latch , 300 mm | 12.00 | Each |
| 12.04 | Front Door stopper | 4.00 | Each |
| 12.05 | Back Door stopper | 4.00 | Each |
| 12.06 | Door Closer of Godrej | 4.00 | Each |
| 13.00 | Providing, laying & testing cast - in - place structural controlled cement concrete grade M25 using 20mm down coarse aggregates but including cost of centering & cement, shuttering and steel reinforcement but including cost of making shear keys and provision of construction / contraction / control joints in various locations, laid, consolidated and cured. Item to include all structural items like base raft, ramp, column, wall, footings, equipment foundations, fins, mullions, beams, lintels, pergola, chajja, facia, parapet, railing, slab trenches, drains and sumps etc. The rates of concrete to include the cost of admixtures / plasticizers where required . This item to also include the concrete requiring waterproofing like water tanks, retaining walls, base raft, etc. On Counters | 0.59 | cum |
| 14.00 | Providing and fixing prepolished 20mm thick stone slab for the toilet counter & facial laid over 12mm thick cement mortar 1:4 (1 cement : 4 coarse sand) inclusive of making and rounding off edges and cutting holes (circular or oval) for wash basin, MS angle to fix bottom stone including rubbing polishing complete including cost of cement. (Basic rate of granite as per SCC - 250 / sq ft) - Counter top/ Facial Dado granite | 7.06 | sqm |
| 15.00 | Providing & fixing of new mirrors of 6mm thick with proper moulding & 4 holes including the SS desginer bolts. Size 2 ft x 2 ft | 10.00 | Each |
| 16.00 | Providing & fixing vitreous china oval (Table top) wash basin 550x400mm dia. for under counter mounting, specially fabricated brackets painted white, 32mm C.P.cast brass bottle trap and pipe to wall with C.P. wall flange and rubber adapter for waste connection complete, including cutting and making good the walls wherever required. |  |  |
| 16.01 | Jaquar Make-OPS-WHT-15601PM | 10.00 | EACH |
| 17.00 | Providing and fixing white vitreous China Indian water closet/ Floor mounted EWC with 100mm / 110mm sand cast iron ‘P‘ or ‘S‘ trap set in cement concrete 1:2:4 mix (1 cement :2 coarse sand :4 graded stone aggregate 20mm nominal size) with 10 litre capacity high or low level PVC cistern with top button or front handle operation along with 32mm dia PVC flush pipe with clamps and rubber adapter joint with C.P. brass screws and washer complete, including cutting and making good the walls and floors etc. Indian type Orissa Pattern size 580x440mm. |  |  |
| 17.01 | Jaquar MakeJCS-WHT-2400 (series) | 8.00 | EACH |
| 18.00 | Providing & fixing 15mm dia C.P. brass bib tap complete including cutting and making good the walls wherever required. |  |  |
| 18.01 | Two way Bib Cock (Jaquar make-ARI-CHR-39041) | 8.00 | EACH |
| 19.00 | Providing & fixing 15mm C.P brass angle valve with C.P. copper connecting pipe 375mm long & nuts, washer and C.P. brass flange complete, including cutting & making good the wall wherever required. (Jaquar Make-AQT-CHR-3057) | 22.00 | EACH |
| 20.00 | Providing & fixing 15mm C.P. pillar tap for W.B.(Jaquar Alive 15 mm Pillar Cock ,ARI-CH-39011) | 10.00 | EACH |
| 21.00 | Providing and fixing Chlorinated polyvinyl Chloride pipes (CPVC) (ASTM F437 & ASTM F438) SDR 13.5 having thermal stability for hot & cold water supply system including all CPVC fitting like plain & brass threaded fittings including cutting and chasing in walls and floors and repairing these in good conditions. This includes jointing pipe & fittings one step with the CPVC solvent cement & testing of joints complete as per direction of the Engineer in-charge |  |  |
| 21.01 | 15 MM | 14.00 | EACH |
| 22.00 | Bottle Trap (Jaquar Make-ALD-CHR-769L250X190) | 16.00 | EACH |
| 23.00 | Washbasin/ Urinal Waste Coupling (Jaquar make-ALD-CHR-709) | 16.00 | EACH |
| 24.00 | PVC Connection Pipe 18 inch (Jaquar make-ALD-CHR-805B) | 24.00 | EACH |
| 25.00 | Providing & fixing new urinal (Jaquar make- URS-WHT-13261/ URS-WHT-13255) of with partitions approved make (Jaquar make-JSE-CHR-810US450X / JSE-CHR-810UC450X (1810-US / 1810-UC)) along with trap, connection to existing drainage line, including testing & finishing. Complete in all respects. | 6.00 | EACH |
| 26.00 | SS chrome polished Bathroom Jali | 24.00 | EACH |
| 27.00 | Health Faucet Set with 1 Mtr. Or 1.5 Mtr. SS Hose. Jaquar make- ALD-CHR-587 | 8.00 | EACH |
| 28.00 | Brass Urinal Jibbi (Spreader)(Jaquar make- ALD-CHR-1043) | 6.00 | EACH |
| 29.00 | Providing & Laying of Waterproofing inside wall including scrubbing of old loose particles (complete plaster) + Dr Fixit chemical SWR latex mixed with cement slurry + laying of Asian paint damp proof course) (2-3 coats), Primer (Oil/Water), Birla putty old surface, Tractor emulsion (2-3 coats), & others. Note: Colour change shall be considered only when a different colour family shall be applied. For the same family no colour change shall be considered. | 55.54 | sqm |
| B | Electrical Works |  |  |
| 1.00 | Providing & fixing 6A modular switch with face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 24 | Each |
| 2.00 | Providing & fixing 6A modular Socket with Shutter, face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 4 | Each |
| 3.00 | Providing & fixing 16A modular switch with face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 4 | Each |
| 4.00 | Providing & fixing 6/16A modular socket with Shutter, face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 4 | Each |
| 5.00 | Providing & fixing LED light fitting (18 W), Surface round/Recessed light as decided by EIC, Make Philips/Wipro | 10 | Each |
| 6.00 | Providing & fixing LED light fitting (10 W), Batten light/Surface as decided by EIC, Make Philips/Wipro | 4 | Each |
| 7.00 | Supply & Installation of Hand dryer for washroom, wall mounted, touchless, drying time 8-10 sec, Make Dolphy/Euronics/jaquar etc as decided by EIC | 4 | Each |

Signature with seal

Annexure

To Whomsoever It May Concern

This is to confirm that we, the undersigned, intend to participate in Tender No. : 2025-26/Estate/2B/LNM-1217 for the tender/work titled: “Renovation of Guest House and ACA Faculty gate Toilet Block”

We hereby certify the following:

No Conflict of Interest:

- We declare that we have no conflict of interest in participating in the above tender. We further confirm that we are not engaged in any activity or association that may compromise, or be perceived to compromise, our impartiality or ability to fulfil the obligations of the tender.

Undertaking on Blacklisting & Litigation (Self-Certificate):

- We hereby undertake that:

- Our agency/firm has never been blacklisted or debarred by any Central Government, State Government, Union Territory administration, or any other Institution/Organization.

- There is no ongoing or past litigation against our agency/firm with any Central Government, State Government, Union Territory administration, or any other Institution/Organization on account of any services/supplies.

- If at any stage the above declaration is found to be false, the Institute shall have the right to take any action as deemed appropriate, including rejection of our bid.

Site Visit /Understanding:

- We confirm that we have visited the site/clearly understood the requirements and expectations of the college authorities.

Scope of Work:

- We have thoroughly reviewed the scope of work mentioned in the bid document. The entire inventory/requirement/work has been verified and found to be in order. We also confirm that no additional items beyond the listed inventory/requirement will be required for successful completion of the work. However, if any additional item is required during execution, the same shall be provided by us as per the requirement.

Bidder Details

Name of Bidder: ______________________________________

Address: _____________________________________________

Contact Person: _______________________________________

Mobile No.: ___________________________________________

Email ID: _____________________________________________

Authorized Signatory: __________________________________

Signature & Stamp: ____________________________________

Date: _________________

Place: ________________

Commercial Bid: (in separate sealed envelope/password protected file via email)

# Bid for the Renovation of Guest House and ACA Faculty gate Toilet Block

# (Tender No. : 2025-26/Estate/2B/LNM-1217)

| Sr. No. | Item Name | QTY. | UOM | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| A | Civil Work |  |  |  |  |
| 1.00 | Dismantling tile work in floors and walls (including removing of Mirrors, glasses, etc.) laid in cement mortar including stacking of serviceable material and disposal of unserviceable material within 50-meter lead: |  |  |  |  |
| 1.01 | For thickness of tiles 25mm to 50mm | 285.57 | sqm |  |  |
| 2.00 | Dismantling doors, windows and clearstory windows steel or wood shutter including chowkhats and holdfasts etc. complete and stacking within 50 meters lead: |  |  |  |  |
| 2.01 | Of area 3 square meter and below. | 12.00 | Each |  |  |
| 3.00 | Dismantling of flushing (WC/EWC) with cistern, urinals of any size including stacking of useful materials near the site and disposal of unserviceable material within 50 meter lead. Urinals & their partions will unitized back with urinal jibbis complete in all respects. |  |  |  |  |
| 3.01 | For thickness of tiles 25mm to 50mm. | 13.00 | Each |  |  |
| 4.00 | Dismantling of light reinforced cement concrete platform along with curtain walls, washbasins, base concrete etc. including stacking of useful materials near the site and disposal of unserviceable materials within 50 metres lead, complete in all respects. |  |  |  |  |
| 4.01 | Above 2.52 Sqm | 5.91 | sqm |  |  |
| 5.00 | Dismantling of old sewage drainage pipes in lavatory & partial shaft (say PVC/CI) including removal, cutting, stacking, disposal within lead 50 metres. Complete in all respects. | 52.08 | metre |  |  |
| 6.00 | Dismantling of existing water supply lines in lavatory & partial shaft (GI/CPVC/UPVC), including valves/fittings/ facuets,etc from riser/trunk including removal, disposal. Complete in all respects. | 53.14 | metre |  |  |
| 7.00 | Providing & laying new sewage drain pipe (e.g. HDPE/PVC- Size 110/75/40 mm) of specified diameter (existing dia, with all necessary fittings - eg. Nani Trap, P-Trap, Sockets, Elbows, etc.) including trenching, bedding, jointing, testing, backfill. | 52.08 | metre |  |  |
| 8.00 | Providing & laying new water supply line (e.g. CPVC/UPVC- Size 20/25/ 32 mm) of specified diameter (existing dia, with all necessary fittings- Tee, Sockets, Elbows, etc.) including fittings, testing, flushing. | 53.14 | metre |  |  |
| 9.00 | Providing and laying Plain Cement Concrete (PCC) of mix 1:4:8 (M7.5) with 40 mm nominal size stone aggregate in foundation and bedding below pipeline, including compacting with plate vibrator, curing, etc., complete in all respects as per the engineers and CPWD/ PWD Norms . | 31.25 | cum |  |  |
| 10.00 | Providing and laying tile flooring of approval make, shade as approved by Architect over adequate thickness of mortar (1:4), making joint with PVC spacers of 3 mm to be filled with tile grout with approved pigmented to match the shade of tile & laid as per design & drawing approved by Architect for various thick of tiles but inculding cost of cement. |  |  |  |  |
| 10.01 | Vitrified tile Somany ( basic rate Rs. 40 per sqft ) (Grande callis brown fp -600 x 1200 mm) | 257.79 | sqm |  |  |
| 10.02 | Vitrified tile Somany (basic rate Rs. 38 per sqft) (Delmon beige - 600 x 600mm) | 55.54 | sqm |  |  |
| 11.00 | Providing and fixing Bijoliya Stone Door frames including hinges jamb, lock Jab, section 110x45mm, or base ties of rigidly fixed together by mechanical means adjustable lugs with mortar guards, lock strike-plate and shock absorbers as specified and applying a coat of approved steel primer & Paint after pre-treatment of the surface as directed. | 66.00 | metre |  |  |
| 12.00 | Providing and fixing 35 mm (overall) thick both side commercial type flush door shutters , core of block board construction and frame of 1st class hardwood phenol bonded, 12mm thick wood lipping on all edges of shutters including 4 nos. brushed steel heavy duty butt hinges 125 mm with matching screws , PVC buffers, polishing where required etc. all complete as per design and drawing. ( The thickness of shutter in all the cases should not be less than 35mm either veneered or laminated finish) (Cost of providing door seal to be included in the quoted rate). | 25.20 | sqm |  |  |
| 12.01 | Providing and fixing tower bolt (barrel type)/ Aldrop SS with necessary screws etc. complete. 250 mm x 10 mm SS | 12.00 | Each |  |  |
| 12.02 | Providing and fixing aluminium handles with necessary screws etc. complete. 150 mm | 12.00 | Each |  |  |
| 12.03 | Providing and fixing sliding door bolts/ latch , 300 mm | 12.00 | Each |  |  |
| 12.04 | Front Door stopper | 4.00 | Each |  |  |
| 12.05 | Back Door stopper | 4.00 | Each |  |  |
| 12.06 | Door Closer of Godrej | 4.00 | Each |  |  |
| 13.00 | Providing, laying & testing cast - in - place structural controlled cement concrete grade M25 using 20mm down coarse aggregates but including cost of centering & cement, shuttering and steel reinforcement but including cost of making shear keys and provision of construction / contraction / control joints in various locations, laid, consolidated and cured. Item to include all structural items like base raft, ramp, column, wall, footings, equipment foundations, fins, mullions, beams, lintels, pergola, chajja, facia, parapet, railing, slab trenches, drains and sumps etc. The rates of concrete to include the cost of admixtures / plasticizers where required . This item to also include the concrete requiring waterproofing like water tanks, retaining walls, base raft, etc. On Counters | 0.59 | cum |  |  |
| 14.00 | Providing and fixing prepolished 20mm thick stone slab for the toilet counter & facial laid over 12mm thick cement mortar 1:4 (1 cement : 4 coarse sand) inclusive of making and rounding off edges and cutting holes (circular or oval) for wash basin, MS angle to fix bottom stone including rubbing polishing complete including cost of cement. (Basic rate of granite as per SCC - 250 / sq ft) - Counter top/ Facial Dado granite | 7.06 | sqm |  |  |
| 15.00 | Providing & fixing of new mirrors of 6mm thick with proper moulding & 4 holes including the SS desginer bolts. Size 2 ft x 2 ft | 10.00 | Each |  |  |
| 16.00 | Providing & fixing vitreous china oval (Table top) wash basin 550x400mm dia. for under counter mounting, specially fabricated brackets painted white, 32mm C.P.cast brass bottle trap and pipe to wall with C.P. wall flange and rubber adapter for waste connection complete, including cutting and making good the walls wherever required. |  |  |  |  |
| 16.01 | Jaquar Make-OPS-WHT-15601PM | 10.00 | EACH |  |  |
| 17.00 | Providing and fixing white vitreous China Indian water closet/ Floor mounted EWC with 100mm / 110mm sand cast iron ‘P‘ or ‘S‘ trap set in cement concrete 1:2:4 mix (1 cement :2 coarse sand :4 graded stone aggregate 20mm nominal size) with 10 litre capacity high or low level PVC cistern with top button or front handle operation along with 32mm dia PVC flush pipe with clamps and rubber adapter joint with C.P. brass screws and washer complete, including cutting and making good the walls and floors etc. Indian type Orissa Pattern size 580x440mm. |  |  |  |  |
| 17.01 | Jaquar MakeJCS-WHT-2400 (series) | 8.00 | EACH |  |  |
| 18.00 | Providing & fixing 15mm dia C.P. brass bib tap complete including cutting and making good the walls wherever required. |  |  |  |  |
| 18.01 | Two way Bib Cock (Jaquar make-ARI-CHR-39041) | 8.00 | EACH |  |  |
| 19.00 | Providing & fixing 15mm C.P brass angle valve with C.P. copper connecting pipe 375mm long & nuts, washer and C.P. brass flange complete, including cutting & making good the wall wherever required. (Jaquar Make-AQT-CHR-3057) | 22.00 | EACH |  |  |
| 20.00 | Providing & fixing 15mm C.P. pillar tap for W.B.(Jaquar Alive 15 mm Pillar Cock ,ARI-CH-39011) | 10.00 | EACH |  |  |
| 21.00 | Providing and fixing Chlorinated polyvinyl Chloride pipes (CPVC) (ASTM F437 & ASTM F438) SDR 13.5 having thermal stability for hot & cold water supply system including all CPVC fitting like plain & brass threaded fittings including cutting and chasing in walls and floors and repairing these in good conditions. This includes jointing pipe & fittings one step with the CPVC solvent cement & testing of joints complete as per direction of the Engineer in-charge |  |  |  |  |
| 21.01 | 15 MM | 14.00 | EACH |  |  |
| 22.00 | Bottle Trap (Jaquar Make-ALD-CHR-769L250X190) | 16.00 | EACH |  |  |
| 23.00 | Washbasin/ Urinal Waste Coupling (Jaquar make-ALD-CHR-709) | 16.00 | EACH |  |  |
| 24.00 | PVC Connection Pipe 18 inch (Jaquar make-ALD-CHR-805B) | 24.00 | EACH |  |  |
| 25.00 | Providing & fixing new urinal (Jaquar make- URS-WHT-13261/ URS-WHT-13255) of with partitions approved make (Jaquar make-JSE-CHR-810US450X / JSE-CHR-810UC450X (1810-US / 1810-UC)) along with trap, connection to existing drainage line, including testing & finishing. Complete in all respects. | 6.00 | EACH |  |  |
| 26.00 | SS chrome polished Bathroom Jali | 24.00 | EACH |  |  |
| 27.00 | Health Faucet Set with 1 Mtr. Or 1.5 Mtr. SS Hose. Jaquar make- ALD-CHR-587 | 8.00 | EACH |  |  |
| 28.00 | Brass Urinal Jibbi (Spreader)(Jaquar make- ALD-CHR-1043) | 6.00 | EACH |  |  |
| 29.00 | Providing & Laying of Waterproofing inside wall including scrubbing of old loose particles (complete plaster) + Dr Fixit chemical SWR latex mixed with cement slurry + laying of Asian paint damp proof course) (2-3 coats), Primer (Oil/Water), Birla putty old surface, Tractor emulsion (2-3 coats), & others. Note: Colour change shall be considered only when a different colour family shall be applied. For the same family no colour change shall be considered. | 55.54 | sqm |  |  |
| B | Electrical Works |  |  |  |  |
|  | Providing & fixing 6A modular switch with face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 24 | Each |  |  |
|  | Providing & fixing 6A modular Socket with Shutter, face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 4 | Each |  |  |
|  | Providing & fixing 16A modular switch with face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 4 | Each |  |  |
|  | Providing & fixing 6/16A modular socket with Shutter, face plate, grid plate, hot dipped galvanised modular box with all other accessories. Make Legrand Mylinc | 4 | Each |  |  |
|  | Providing & fixing LED light fitting (18 W), Surface round/Recessed light as decided by EIC, Make Philips/Wipro | 10 | Each |  |  |
|  | Providing & fixing LED light fitting (10 W), Batten light/Surface as decided by EIC, Make Philips/Wipro | 4 | Each |  |  |
|  | Supply & Installation of Hand dryer for washroom, wall mounted, touchless, drying time 8-10 sec, Make Dolphy/Euronics/jaquar etc as decided by EIC | 4 | Each |  |  |
| Total |  |  |  |  |  |
| GST |  |  |  |  |  |
| Total including GST |  |  |  |  |  |

GENERAL TERMS AND CONDITIONS:

- All applicable taxes such as GST must be clearly mentioned. If not specified, it shall be deemed that all such charges are included in the quoted price.

- If the supplier does not have a particular item or proposes a different make than that specified in the tender, the same must be clearly highlighted in the bid.

- Prices must be inclusive of all charges such as packing, forwarding, transit insurance, transportation, loading, unloading (up to 50 meters), installation, and any other incidental expenses.

- Bid must be submitted on an F.O.R. basis for delivery at: The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur – 302031.

- Bid shall remain valid for a minimum period of 30 days. No change in prices will be accepted during the validity period.

- The LNMIIT reserves the right to reject any quotation without assigning any reason.

- Payment terms shall be mutually decided and agreed upon between the Institute and the vendor.

- The bidder must submit a No Conflict of Interest Declaration along with the bid.

- The bid must be submitted on the company’s letterhead and in the LNMIIT-prescribed format only.

- Any item or service not being supplied by the bidder must be clearly mentioned separately in the bid.

- Vendors must provide a photocopy of their cheque/cancelled cheque along with the invoice for payment processing.

- Prices shall be quoted in Indian Rupees (INR) only.

- Total prices shall be quoted for the complete scope of work as per technical specifications, inclusive of all taxes, duties, insurance, packing, forwarding, and incidental charges.

- Agencies or individuals who are debarred or blacklisted are not permitted to participate in the tendering process. Any agency engaging a debarred person for negotiations or representation is also liable to be debarred.

- If the work is not completed within the stipulated time mentioned in the work order, the competent authority of LNMIIT reserves the right to accept partial work on suitable terms, cancel the supply/work order, withhold or stop payment, forfeit the Earnest Money Deposit, or debar the agency. The decision of the Competent Authority shall be final and binding.

- In case two or more bidders quote the same rates, the final selection shall be made through a draw of lots conducted by a committee comprising LNMIIT administrative members(PCC).

- Vendors are required to visit the site and conduct a detailed survey before submitting their bid to understand the exact requirements. No request for additional items or modifications will be entertained later.

- A pre-bid meeting and/or site visit may be arranged prior to submission of bids to clarify any doubts regarding project details, scope of work, and required documents.

Signature with seal