# Tender No:. 2025-26/ESTATE/2B/LNM-1232

The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non-Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposals from competent vendors for “SITC of Street Light”. Bidders can submit their sealed offers/s at the following address by Email/Speed-Post/ Registered Post/ By-hand/courier before 05:00 PM on 14.03.2026 (Last date of bid submission).

Filing of Tender:

Option 1: Physical submission of offers (hard copy) in sealed envelopes.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the institute in person/ Speed-Post/ Registered Post/ By-hand/courier of the Institute after marking due entries in the Receipt Book available with the Security Officer at the main gate.

The tender subject and reference number must be mentioned on the envelope.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Option 2: Password-protected online submission via Email

Password-protected offers can be submitted by the bidders via email at the below-mentioned email IDs.

tender-document@lnmiit.ac.in (password-protected document)

tender-password@lnmiit.ac.in (password only)

The tender subject and reference number must be mentioned in the subject line of the emails.

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory, are required to be submitted in two separate envelopes/emails.

Envelopes/Emails should be duly marked as follows:

- Technical bid for the: SITC of Street Light

- Commercial Bid for the: SITC of Street Light

- Both Envelopes/files should be enclosed/attached in a single envelope/email bearing the heading “Bid for SITC of Street Light” (Tender No. : 2025-26/Estate/2B/LNM-1232)”.

Bids are liable to be rejected:

- If the above-mentioned bid submission procedure is not followed.

- Incomplete and non-conforming bids.

- Arbitration: All disputes and differences arising between the two parties in connection with this order shall be mutually settled as far as possible, failing which, all such disputes shall be referred to arbitration and settlement under the provision of the Arbitration and Conciliation Act, 1996 and as amended from time to time or any such law for the time being prevailing in India. The venue of such arbitration shall be Jaipur.

Technical Bid: (in a separate sealed envelope/via email)

# Bid for SITC of Street Light

# (Tender No. : 2025-26/Estate/2B/LNM-1232)

Bidder is required to furnish the requisite details as per the below mentioned table format and proof of such documents to be uploaded along with the bid under respective title/ headings as mentioned at column no ‘C’ accordingly.

| S.No. | Title/ Heading Details sought by buyer | Details/ Certificate No. (Attached Y/N) |
| --- | --- | --- |
| A | B | C |
| 1. | Bidder Firm Name & Registration No, |  |
| 2. | Bidder’s Company profile or brief note |  |
| 3. | PAN Card No,. |  |
| 4. | GST Number |  |
| 5. | List of Directors/ Partners/Proprietor/ Members of the Bidder’s Firm |  |
| 6. | Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members |  |
| 7. | Bidder Turnover - Average annual turnover for last three years |  |
| 8. | The Client List with some purchase orders and satisfactory certificate (to whom the vendor has delivered the goods/provided the services in the past (Three (03) purchase orders/work orders, at least one from each of the last three years, including the previous year.) |  |
| 9. | If the bidder is authorized by the OEM/Manufacturer. Proof of the same is to be submitted with the Technical Bid |  |
| 10. | An undertaking (self-certificate) that the agency has not been blacklisted by a Central/State/UT Government Institutions and there has been no litigation with any government department/institution on account of any services/supplies. |  |
| 11. | Scanned copy of EMD/ Tender Fees paid through RTGS/NEFT (if required/asked in the tender) |  |
| 12 | Xerox copies of MoUs with previous colleges (minimum three) must be attached. (Applicable for student fest only). |  |

- Technical specification:

# SCOPE OF WORK
SITC OF STREET LIGHT SYSTEM

1. GENERAL
This document defines the detailed scope of work for Supply, Installation, Testing and Commissioning (SITC) of Street Lighting System as per approved BOQ, drawings, specifications, and directions of Engineer-in-Charge.

- SCOPE OF WORK
The contractor shall supply all materials, labour, tools, tackles, equipment, transportation, handling, testing instruments and safety arrangements required for completion of the work.

2.1 Street Light Pole
- Supply, installation, testing and commissioning of 3.0-meter hot dip galvanized octagonal GI pole.
- Zinc coating minimum 65 microns.
- Base plate Fe-410 as per IS 2062 with four foundation bolts.
- Lockable inspection door with DP MCB and terminal connectors.
- Complete erection and alignment on RCC foundation.

2.2 Overhang / Bracket
- OEM fabricated hot dip galvanized MS overhang.
- Pipe size 48.3 x 3.25 mm.
- Single / Double arm as per BOQ.
- Proper welding, alignment and finishing.

2.3 RCC Foundation
- Excavation of pit up to required depth.
- Dry stone pitching 200 mm thick.
- Providing PLB HDPE pipes and foundation bolts with template.
- RCC M-25 footing (700x700x250 mm) and pedestal (300x300x450 mm).
- Proper curing and disposal of surplus earth.

2.4 Junction Box
- Supply and fixing of waterproof, weatherproof PVC junction box.
- With 16A connector and DP MCB.
- Make Suntex or equivalent.

2.5 Cable Supply
- XLPE insulated, PVC sheathed armoured copper cable.
- Conforming to IS 1554 / IS 7098.
- Sizes: 4 sqmm 4 core, 1.5 sqmm 3 flexible core.

2.6 Cable Laying on Wall
- Laying XLPE cable directly on wall.
- Proper clamping and protection.

2.7 Underground Cable Laying
- Trench excavation 75 cm depth x 35 cm width.
- Sand bedding, warning tape and backfilling.
- As per Indian standards.

2.8 Testing & Commissioning
- Insulation resistance test.
- Continuity and polarity test.
- Functional testing and trial run.

- CODES & STANDARDS
All works shall comply with latest IS codes, CPWD specifications and local electricity authority norms.

- SAFETY
Contractor shall follow all safety norms and be responsible for any accident or damage.

- MEASUREMENT & PAYMENT
- Measurement shall be as per BOQ.
- Payment shall be made on actual measured quantities.
- Rates inclusive of all taxes and duties.

- DEFECT LIABILITY
Defect liability period shall be 12 months from commissioning. Any fault identified during this period shall be rectified by vendor free of charge.

- COMPLETION TIME
Work shall be completed within stipulated time as per work order.

- Work must be executed as per Engineer in charge.

| S. No. | DESCRIPTION | UNIT | Qty. |
| --- | --- | --- | --- |
|  | SUPPLY, INSTALLATION, TESTING & COMMISSIONING (SITC) OF STREET LIGHT |  |  |
| 1 | SITC of 3 Mtr Galvanized Iron pole with galvanizing done in single dipping (Average coating thickness Minimum 65 Microns) continuously tapered poles having Octagonal sections designed pole shafts shall be provided with the rigid flange plate of a suitable thickness (Fe 410 conforming to IS 226 / IS 2062) with provision for fixing 4 no. foundation bolts, This base plate shall be fillet welded to the pole shaft at two locations i.e. from inside and outside. The octagonal Poles shall have a lockable door of approximately 500 mm in length at the elevation of 500 mm from the Base plate with a bakelite sheet having 6A/10A DP MCB and 16 sqm stud type connector ( 4 nos) inside the pole at the door opening for cable connection of following length and dimension with base plate, anchor plate and foundation bolt on the cement concrete foundation complete in all aspect. | Nos. | 15.00 |
| 2 | Supply and Fixing of Pole OEM fabricated hot dipped galvanised M.S. Overhang (48.3 X 3.25 mm) with cap (400 x 88.9 / 114.3 x 3.25 mm) over the existing poles. All as per pre-approved by Engineer in charge. |  |  |
| a | Double arm overhang | Nos. | 15.00 |
| 2 | Providing and Laying RCC Foundation for Octagonal pole as per pre-approved by Engineer in charge. | Nos. | 15.00 |
|  | a) Excavation of Pit for Foundation Size upto required depth from Ground level |  |  |
|  | b) Dressing of Pit and Dry-stone pitching 200 mm thick including supply of stones and preparing surface complete. |  |  |
|  | c) Providing & fixing of 2 no. Required Size PLB HDPE Pipe & Foundation Bolt with template |  |  |
|  | d) Providing and Laying Reinforce cement Concrete of M-25 grade in Footing and Pedestal including Curing Footing (700mm x 700mm x 250mm) Pedestal (300mm x 300mm x 450mm) |  |  |
| 3 | Supply of PVC waterproof, weather proff, rust proof & termite resistance junction box with 16A capacity backlite connector & DIL rain for DP. Make Suntex or equivalent as decided by EIC. | Nos. | 15.00 |
| 4 | Supplying XLPE insulated & P.V.C. sheathed Armoured Copper cable confirming to IS:1554 P-I / IS :7098 P - I of 1.1 KV with electrolytic grade Copper made of copper rods confirming to IS 613-1964 of purity >99.7 %, round / flat strip armouring, Inner / outer sheath confirming to IS:5831 |  |  |
| a | 4.0 Sq. mm 4 core | Mtr. | 270 |
| 5 | Supply of 1.5 Sqmm 3 core flexiable cable | Mtr. | 140 |
| 6 | Laying of one number XLPE power cable of 1.1 KV grade direct on wall |  |  |
| 6.1 | 4.0 Sq. mm 4 core | Mtr. | 260 |
| 7 | Laying of one number XLPE power cable of 1.1 KV grade direct in ground including excavation of trench of size 75 cm (Depth) x 35 cm (Width) , with procedure as per indian standard. |  |  |
| 7.1 | 3C x 1.5 Sqmm | Mtr. | 140 |

Annexure

To Whomsoever It May Concern

This is to confirm that we, the undersigned, intend to participate in Tender No. : 2025-26/Estate/2B/LNM-1232 (Tender Number) for the tender/work titled: “SITC of Street Light” (Item Name)”

We hereby certify the following:

No Conflict of Interest:

- We declare that we have no conflict of interest in participating in the above tender. We further confirm that we are not engaged in any activity or association that may compromise, or be perceived to compromise, our impartiality or ability to fulfil the obligations of the tender.

Undertaking on Blacklisting & Litigation (Self-Certificate):

- We hereby undertake that:

- Our agency/firm has never been blacklisted or debarred by any Central Government, State Government, Union Territory administration, or any other Institution/Organization.

- There is no ongoing or past litigation against our agency/firm with any Central Government, State Government, Union Territory administration, or any other Institution/Organization on account of any services/supplies.

- If at any stage the above declaration is found to be false, the Institute shall have the right to take any action as deemed appropriate, including rejection of our bid.

Site Visit /Understanding:

- We confirm that we have visited the site/clearly understood the requirements and expectations of the college authorities.

Scope of Work:

- We have thoroughly reviewed the scope of work mentioned in the bid document. The entire inventory/requirement/work has been verified and found to be in order. We also confirm that no additional items beyond the listed inventory/requirement will be required for successful completion of the work. However, if any additional item is required during execution, the same shall be provided by us as per the requirement.

Bidder Details

Name of Bidder: ______________________________________

Address: _____________________________________________

Contact Person: _______________________________________

Mobile No.: ___________________________________________

Email ID: _____________________________________________

Authorized Signatory: __________________________________

Signature & Stamp: ____________________________________

Date: _________________

Place: ________________

Commercial Bid: (in separate sealed envelope/password protected file via email)

# Bid for SITC of Street Light

# (Tender No. : 2025-26/Estate/2B/LNM-1232)

| S. No. | DESCRIPTION | UNIT | Qty. | RATE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | SUPPLY, INSTALLATION, TESTING & COMMISSIONING (SITC) OF STREET LIGHT |  |  |  |  |
| 1 | SITC of 3 Mtr Galvanized Iron pole with galvanizing done in single dipping (Average coating thickness Minimum 65 Microns) continuously tapered poles having Octagonal sections designed pole shafts shall be provided with the rigid flange plate of a suitable thickness (Fe 410 conforming to IS 226 / IS 2062) with provision for fixing 4 no. foundation bolts, This base plate shall be fillet welded to the pole shaft at two locations i.e. from inside and outside. The octagonal Poles shall have a lockable door of approximately 500 mm in length at the elevation of 500 mm from the Base plate with a Bakelite sheet having 6A/10A DP MCB and 16 sqm stud type connector ( 4 no’s) inside the pole at the door opening for cable connection of following length and dimension with base plate, anchor plate and foundation bolt on the cement concrete foundation complete in all aspect. | Nos. | 15.00 |  |  |
| 2 | Supply and Fixing of Pole OEM fabricated hot dipped galvanised M.S. Overhang (48.3 X 3.25 mm) with cap (400 x 88.9 / 114.3 x 3.25 mm) over the existing poles. All as per pre-approved by Engineer in charge. |  |  |  |  |
| a | Double arm overhang | Nos. | 15.00 |  |  |
| 2 | Providing and Laying RCC Foundation for Octagonal pole as per pre-approved by Engineer in charge. | Nos. | 15.00 |  |  |
|  | a) Excavation of Pit for Foundation Size up to required depth from Ground level |  |  |  |  |
|  | b) Dressing of Pit and Dry-stone pitching 200 mm thick including supply of stones and preparing surface complete. |  |  |  |  |
|  | c) Providing & fixing of 2 no. Required Size PLB HDPE Pipe & Foundation Bolt with template |  |  |  |  |
|  | d) Providing and Laying Reinforce cement Concrete of M-25 grade in Footing and Pedestal including Curing Footing (700mm x 700mm x 250mm) Pedestal (300mm x 300mm x 450mm) |  |  |  |  |
| 3 | Supply of PVC waterproof, weatherproof, rust proof & termite resistance junction box with 16A capacity backlite connector & DIL rain for DP. Make Suntex or equivalent. | Nos. | 15.00 |  |  |
| 4 | Supplying XLPE insulated & P.V.C. sheathed Armoured Copper cable confirming to IS:1554 P-I / IS :7098 P - I of 1.1 KV with electrolytic grade Copper made of copper rods confirming to IS 613-1964 of purity >99.7 %, round / flat strip armouring, Inner / outer sheath confirming to IS:5831 |  |  |  |  |
| a | 4.0 Sq. mm 4 core | Mtr. | 270 |  |  |
| 5 | Supply of 1.5 Sqmm 3 core flexible cable | Mtr. | 140 |  |  |
| 6 | Laying of one number XLPE power cable of 1.1 KV grade direct on wall |  |  |  |  |
| 6.1 | 4.0 Sq. mm 4 core | Mtr. | 260 |  |  |
| 7 | Laying of one number XLPE power cable of 1.1 KV grade direct in ground including excavation of trench of size 75 cm (Depth) x 35 ccm (width) , with procedure as per Indian standard. |  |  |  |  |
| 7.1 | 3C x 1.5 Sqmm | Mtr. | 140 |  |  |
| Amount |  |  |  |  |  |
| GST as Applicable |  |  |  |  |  |
| Amount Including GST |  |  |  |  |  |

GENERAL TERMS AND CONDITIONS:

- All applicable taxes such as GST must be clearly mentioned. If not specified, it shall be deemed that all such charges are included in the quoted price.

- If the supplier does not have a particular item or proposes a different make than that specified in the tender, the same must be clearly highlighted in the bid.

- Prices must be inclusive of all charges such as packing, forwarding, transit insurance, transportation, loading, unloading (up to 50 meters), installation, and any other incidental expenses.

- Bid must be submitted on an F.O.R. basis for delivery at: The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur – 302031.

- Bid shall remain valid for a minimum period of 30 days. No change in prices will be accepted during the validity period.

- The LNMIIT reserves the right to reject any quotation without assigning any reason.

- Payment terms shall be mutually decided and agreed upon between the Institute and the vendor.

- The bidder must submit a No Conflict of Interest Declaration along with the bid.

- The bid must be submitted on the company’s letterhead and in the LNMIIT-prescribed format only.

- Any item or service not being supplied by the bidder must be clearly mentioned separately in the bid.

- Vendors must provide a photocopy of their cheque/cancelled cheque along with the invoice for payment processing.

- Prices shall be quoted in Indian Rupees (INR) only.

- Total prices shall be quoted for the complete scope of work as per technical specifications, inclusive of all taxes, duties, insurance, packing, forwarding, and incidental charges.

- Agencies or individuals who are debarred or blacklisted are not permitted to participate in the tendering process. Any agency engaging a debarred person for negotiations or representation is also liable to be debarred.

- If the work is not completed within the stipulated time mentioned in the work order, the competent authority of LNMIIT reserves the right to accept partial work on suitable terms, cancel the supply/work order, withhold or stop payment, forfeit the Earnest Money Deposit, or debar the agency. The decision of the Competent Authority shall be final and binding.

- In case two or more bidders quote the same rates, the final selection shall be made through a draw of lots conducted by a committee comprising LNMIIT administrative members(PCC).

- Vendors are required to visit the site and conduct a detailed survey before submitting their bid to understand the exact requirements. No request for additional items or modifications will be entertained later.

- A pre-bid meeting and/or site visit may be arranged prior to submission of bids to clarify any doubts regarding project details, scope of work, and required documents.

Signature with seal

Signature with seal