The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposal from competent vendors for “Wooden work in Laboratory (Physics Lab R.No. 2062-63)”. Bidders can submit their sealed offer/s at the following address by Speed-Post/ Registered Post/ By-hand (but not by Courier)/ before 05:00 PM on 26.02.2023.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the Institute after marking due entries.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Email: directoroffice@lnmiit.ac.in

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory are required to be submitted in two separate envelopes/files. Both envelopes/files should be duly marked as “Technical bid for “Wooden work in Laboratory (Physics Lab R.No. 2062-63)”.” and “Commercial Bid for “Wooden work in Laboratory (Physics Lab R.No. 2062-63)”.” respectively. Both Envelopes shall be enclosed in a single envelope bearing the heading “Bid for “Wooden work in Laboratory (Physics Lab R.No. 2062-63)”.”. Bids are liable to be rejected if any or both of the technical or commercial offers are found in open condition or required information is not found with the offer.

Technical Bid: (in a separate sealed envelope)

While submitting quotations, the following procedure/ formalities and conditions may kindly be adhered.

- Company profile or brief note about the bidder's company, PAN number, and GST number should be submitted by the Vendor.

- The List of Directors/Partners/Proprietor/Members of the Firm to be attached.

- Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members to be provided.

- Turn over for the last three years to be mentioned. (Copy to be enclosed)

- List of the client to whom the vendor has delivered the goods or provided the services in past.

- Specification as mentioned below, shall be provided as per the details given below:

Scope of work, Technical data sheet and Specification of the item/s

Bid for “Wooden work in Laboratory (Physics Lab R.No. 2062-63).”

| S.No. | Item Name/ Description | Qty. | UOM |
| --- | --- | --- | --- |
| 1 | Providing & fixing 19 MM block board with teak wood linings and moldings, frame (With Melamine polish) in furniture like office table, Computer table, Almira. Wooden cabinet, Shelves etc. including cutting fitting all type approved fittings, adhesive and accessories like lock, telescopic channel, hinge as per directed by user/senior engineer In-charge. Make- Century ply, Green Ply. Edges: - All edges are covered with PVC edge bands (2 mm thick) or Wooden Moulded (Painted). As per IS: 1659, Type: COM, Termite resistant, Fire resistant, Moisture repulant and Y.O.M.:2022 Hardware (Locks, keys, handles, channels, hinges, and other accessories. - Moda/haffle/Hetich/Ebco. Keyboards for 13/14 seats with cable manager & footrest. |  |  |
| 1.1 | Both side Laminate/ Single side or enamel paint (Make- Green laminate) | 650.00 | SqFt. |
| 1.2 | Both side filling material putty, two or more coat wooden primer & two or more coat synthetic high gloss enamel paint. Make Asian paints. | 221.00 | SqFt. |
| 2 | Providing & fixing 6 MM wooden ply with filling material putty, two or more coat wooden primer & two or more coat synthetic high gloss enamel paint in furniture like office table t, Computer table, Almira. Wooden cabinet, Shelves etc. including cutting fitting all type approved fittings, adhesive and accessories like lock, telescopic channel, hinge as per directed by user/senior engineer In-charge. Make- Century ply, Green Ply. | 162.00 | SqFt. |
| 3 | Dismenling & removing refitting as per existing condition old wooden almira set as directed new location including lift & complete lead in the campus premises. | 1.00 | Set/ job |
|  | NOTE:- Pictures are for reference only. Actual things may move according to the user. |  |  |

Room No- 2063 (Computer Table Set Pics-1

Room No- 2063 (Computer Table Set Pics-2

Room No- 2062 (Cabinet under work station)

Commercial Bid: (in separate sealed envelope)

Bid for “Wooden work in Laboratory (Physics Lab R.No. 2062-63)”.

| S.No. | Item Name/ Description | Qty. | UOM | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Providing & fixing 19 MM block board with teak wood linings and moldings, frame (With Melamine polish) in furniture like office table, Computer table, Almira. Wooden cabinet, Shelves etc. including cutting fitting all type approved fittings, adhesive and accessories like lock, telescopic channel, hinge as per directed by user/senior engineer In-charge. Make- Century ply, Green Ply. Edges: - All edges are covered with PVC edge bands (2 mm thick) or Wooden Moulded (Painted). As per IS: 1659, Type: COM, Termite resistant, Fire resistant, Moisture repulant and Y.O.M.:2022 Hardware (Locks, keys, handles, channels, hinges, and other accessories. - Moda/haffle/Hetich/Ebco. Keyboards for 13/14 seats with cable manager & footrest. |  |  |  |  |
| 1.1 | Both side Laminate/ Single side or enamel paint (Make- Green laminate) | 650.00 | SqFt. |  |  |
| 1.2 | Both side filling material putty, two or more coat wooden primer & two or more coat synthetic high gloss enamel paint. Make Asian paints. | 221.00 | SqFt. |  |  |
| 2 | Providing & fixing 6 MM wooden ply with filling material putty, two or more coat wooden primer & two or more coat synthetic high gloss enamel paint in furniture like office table t, Computer table, Almira. Wooden cabinet, Shelves etc. including cutting fitting all type approved fittings, adhesive and accessories like lock, telescopic channel, hinge as per directed by user/senior engineer In-charge. Make- Century ply, Green Ply. | 162.00 | SqFt. |  |  |
| 3 | Dismenling & removing refitting as per existing condition old wooden almira set as directed new location including lift & complete lead in the campus premises. | 1.00 | Set/ Job |  |  |
|  | NOTE:- Pictures are for reference only. Actual things may move according to the user. |  |  |  |  |

- Taxes like GST shall be clearly mentioned. If nothing is mentioned, these charges shall be deemed included.

- If the supplier does not have a particular item offered in the tender or a different make is being proposed by him, the same should be highlighted/mentioned especially by him.

- The prices should be including all (Packing, forwarding, Transit Insurance, Transportation Loading/unloading, Installation etc.). Loading/unloading up to 50 Mtr. shall be done by the supplier.

- Bid should be submitted for F.O.R delivery at the destination i.e. The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur- 302031

- Bid should be valid for a minimum period of 30 days. The Change in prices shall not be accepted within the validity period.

- The LNMIIT reserves the right to reject any quotation(s) without assigning any reasons.

- Payment terms shall be decided mutually.

- There should not be any conflict of interest (Declaration to be submitted by the Vendor).

- Bid should be on the company’s letterhead and should submit their bid in the LNMIIT format only

- Item/Service not supplied by the supplier/service provider should be mentioned separately.

- Vendor should provide a photocopy of the cheque/cancelled cheque while presenting the Invoice to LNMIIT.

- Vendor shall provide last 3 years technical documents (work orders, purchase orders, company profile, work completion certificates, company office and godam picture and address, & etc.). In addition, if possible then provide financial record of last 3 years to financial round.

- Vendor shall take care all the work by themselves, no LNMIIT person shall responsible for any casualty.

- Pre-bid meeting and pre-bid site visit can be done before quoting the rate in the tender to clear up any confusion regarding project details, the scope of work, and solicitation of documents.

All charges applicable shall be clearly mentioned with the offer. If nothing is mentioned in the offer, the quoted price shall be treated as final, and nothing extra shall be paid extra.

Signature with seal