The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposals from competent vendors for “Supply and installation of chemical filled earthing electrode work with complete accessories”. Bidders can submit their sealed offer/s at the following address by Speed-Post/ Registered Post/ By-hand (but not by Courier)/ before 05:00 PM on 05.03.2023.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the Institute after marking due entries.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Email: directoroffice@lnmiit.ac.in

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory are required to be submitted in two separate envelopes/files. Both envelopes/files should be duly marked as “Technical bid for “Supply and installation of chemical filled earthing electrode work with complete accessories” and “Commercial Bid for “Supply and installation of chemical filled earthing electrode work with complete accessories” respectively. Both Envelopes shall be enclosed in a single envelope bearing the heading “Bid for Supply and installation of chemical filled earthing electrode work with complete accessories”. Bids are liable to be rejected if any or both of the technical or commercial offers are found in open condition or required information is not found with the offer.

Technical Bid: (in a separate sealed envelope)

While submitting quotations, the following procedure/ formalities and conditions may kindly be adhered.

- Company profile or brief note about the bidder's company, PAN number, and GST number should be submitted by the Vendor.

- The List of Directors/Partners/Proprietor/Members of the Firm to be attached.

- Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members to be provided.

- Turn over for the last three years to be mentioned. (Copy to be enclosed)

- List of the client to whom the vendor has delivered the goods or provided the services in past.

- Specification as mentioned below, shall be provided as per the details given below:

Scope of work, Technical data sheet and Specification of the item/s

| Feeder Pillars Panel _ Earthling Work |  |  |  |
| --- | --- | --- | --- |
| S.NO. | DESCRIPTION OF ITEM | UNIT | Qty. |
|  | Supply & Installation of LT Panel Feeder Pillars Earthling Work |  |  |
| A | EARTHING |  |  |
| 1 | Supplying & Grounding Maintencen free chemical filled earthling electrode 48mm x 3000mm including accessories & providing masonry enclosure with cover plate locking arrangement complete as required. ( Cover Plate Size : 300 x 300 MM ) | Set | 20 |
| 2 | Providing, laying in ground and on surface with saddles at required intervals and testing of 25 X 6 mm size of GI strip, including jointing of GI strips with Nuts and Bolts at required locations. | Meter | 400 |
| 3 | Providing and fixing PVC shrinkable sleeve of required size for GI strip of size 25 X 6 mm, for earting strips from Transformer to Pit. | Meter | 200 |
| 4 | Supply and installation of 1000 mm wide 25 mm thick rubber matting of approved make as per electricity rules .(HT Side) | Meter | 15 |
| B | Miscellaneous |  |  |
| 1 | Providing and laying 250 mm dia Class III uPVC pipe, after cutting and dismantling of PCC and digging upto the level up to existing trench in Sub-Station, and back filling, repairing and making good as required, for laying of LT cable from DG foundation to LT Panel in Sub-Station. | Meter | 20 |

| Earthling work for Feeder Panel Details |  |  |  |
| --- | --- | --- | --- |
| S.No. | PANEL LOCATION | Type of Panel | No. of panel |
| 1 | Girl Hostel - A/B/C | Feeder Panel (L & P ) | 1 |
|  |  | Feeder PANEL ( HVAC) | 1 |
| 2 | Nearby Director House | Street Lights Panel | 1 |
| 3 | Near By Ghuset House | Street Lights Panel | 1 |
| 4 | BH - 1 back side nearby B/C/D - Type Quarter | Feeder Panel (L & P ) | 1 |
|  |  | Feeder PANEL ( HVAC) | 1 |
| 5 | Nearby C- Type Quarter | Feeder PANEL ( HVAC) | 1 |
| 6 | Studio Apartment Type Quarter | Feeder Panel (L & P ) | 1 |
| 7 | Nearby Temple | Feeder Panel (L & P ) | 1 |
|  |  | Feeder PANEL ( HVAC) | 1 |
| 8 | Boy Hostel - III | Feeder Panel (L & P ) | 1 |
|  |  | Feeder PANEL ( HVAC) | 1 |
|  |  | Street Lights Panel | 1 |
| 9 | Nearby Boy Hostel - II | Feeder PANEL ( HVAC) | 1 |
| 10 | Near By Lecture Hall 9 & 10 | Street Lights Panel | 1 |
| 11 | Food Court Back Side | Feeder Panel (L & P ) | 1 |
|  |  | Total No. of Feeder Panels | 16 |

Commercial Bid: (in separate sealed envelope)

Bid for “Supply and installation of chemical filled earthing electrode work with complete accessories”

| Feeeder Pillars Panel _ Earthing Work |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S.NO. | DESCRIPTION OF ITEM |  | UOM |  |  |
|  | Supply & Installation of LT Panel Feeder Pillars Earthing Work | Qty. |  | Rate | Amount |
| A | EARTHING |  |  |  |  |
| 1 | Supplying & Grounding Maintance free chemical filled earthing electrode 48mm x 3000mm including accessories & providing masonry enclosure with cover plate locking arrangement complete as required. ( Cover Plate Size : 300 x 300 MM ) | 20 | Set |  |  |
| 2 | Providing, laying in ground and on surface with saddles at required intervals and testing of 25 X 6 mm size of GI strip, including jointing of GI strips with Nuts and Bolts at required locations. | 400 | Meter |  |  |
| 3 | Providing and fixing PVC shrinkable sleeve of required size for GI strip of size 25 X 6 mm, for earting strips from Transformer to Pit. | 200 | Meter |  |  |
| 4 | Supply and installation of 1000 mm wide 25 mm thick rubber matting of approved make as per electricity rules .(HT Side) | 15 | Meter |  |  |
| B | Miscellaneous |  |  |  |  |
| 1 | Providing and laying 250 mm dia Class III uPVC pipe, after cutting and dismantling of PCC and digging upto the level upto existing trench in Sub-Station, and back filling, repairing and making good as required, for laying of LT cable from DG foundation to LT Panel in Sub-Station. | 20 | Meter |  |  |

| Total |  |
| --- | --- |
| GST |  |
| Total including GST |  |

- Taxes like GST shall be clearly mentioned. If nothing is mentioned, these charges shall be deemed included.

- If the supplier does not have a particular item offered in the tender or a different make is being proposed by him, the same should be highlighted/mentioned especially by him.

- The prices should be including all (Packing, forwarding, Transit Insurance, Transportation Loading/unloading, Installation etc.). Loading/unloading up to 50 Mtr. shall be done by the supplier.

- Bid should be submitted for F.O.R delivery at the destination i.e. The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur- 302031

- Bid should be valid for a minimum period of 30 days. The Change in prices shall not be accepted within the validity period.

- The LNMIIT reserves the right to reject any quotation(s) without assigning any reasons.

- Payment terms shall be decided mutually.

- There should not be any conflict of interest (Declaration to be submitted by the Vendor).

- Bid should be on the company’s letterhead and should submit their bid in the LNMIIT format only

- Item/Service not supplied by the supplier/service provider should be mentioned separately.

- Vendor should provide a photocopy of the cheque/cancelled cheque while presenting the Invoice to LNMIIT.

- Pre-bid meeting and pre-bid site visit can be done before quoting the rate in the tender to clear up any confusion regarding project details, the scope of work, and solicitation of documents.

All charges applicable shall be clearly mentioned with the offer. If nothing is mentioned in the offer, the quoted price shall be treated as final, and nothing extra shall be paid extra.

Signature with seal