The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposals from competent vendors for “Supply and installation work for repair and maintenance of Air Cooling Machines”. Bidders can submit their sealed offer/s at the following address by Speed-Post/ Registered Post/ By-hand (but not by Courier)/ before 05:00 PM on 05.03.2023.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the Institute after marking due entries.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Email: directoroffice@lnmiit.ac.in

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory are required to be submitted in two separate envelopes/files. Both envelopes/files should be duly marked as “Technical bid for “Supply and installation work for repair and maintenance of Air Cooling Machines” and “Commercial Bid for “Supply and installation work for repair and maintenance of Air Cooling Machines” respectively. Both Envelopes shall be enclosed in a single envelope bearing the heading “Bid for Supply and installation work for repair and maintenance of Air Cooling Machines”. Bids are liable to be rejected if any or both of the technical or commercial offers are found in open condition or required information is not found with the offer.

Technical Bid: (in a separate sealed envelope)

While submitting quotations, the following procedure/ formalities and conditions may kindly be adhered.

- Company profile or brief note about the bidder's company, PAN number, and GST number should be submitted by the Vendor.

- The List of Directors/Partners/Proprietor/Members of the Firm to be attached.

- Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members to be provided.

- Turn over for the last three years to be mentioned. (Copy to be enclosed)

- List of the client to whom the vendor has delivered the goods or provided the services in past.

- Specification as mentioned below, shall be provided as per the details given below:

Scope of work, Technical data sheet and Specification of the item/s

| LNM INSTITUTE FOR INFORMATION AND TECHNOLOGY,JAMDOLI, JAIPUR |  |  |  |
| --- | --- | --- | --- |
| NAME OF WORK : Center Air Cooling Maintenance Work |  |  |  |
| PLACE OF WORK : Hostels |  |  |  |
| S.no. | DESCRIPTION | UNIT | QTY. |
|  | AIRWASHER MACHINES : |  |  |
| 1 | Supply, fabrication, installation and testing of sheet metal ducts in accordance with the approved shop drawing and as per specifications. MAKE -Tata/ESSAR/ JINDAL |  |  |
| a | 22 G (0.80mm) galvanized sheet metal steel | Sqft | 350 |
| b | 24 G (0.63mm) galvanized sheet metal steel | Sqft | 256 |
| c | 22 G galvanized sheet metal steel (with MS flange) | Sqft | 150 |
| 2 | 15000 CFM AIRWASHER MACHINES : New body |  |  |
|  | Supply,installation, testing & commisioning of double skin package type air washer of minimum 90% efficiency with imported centrifugal fan of static pressure 50 mm of WG, motor, mounting arrangements, metallic vibration isolation, belt drive arrangement, VCD, distribution headers, access door, pump, stainless steel water tank, suction screen, imported cellulose paper fills, prefilter, GI B’ class piping & associated works, quick fill, makeup & drain connections with ball valves & float valves etc complete.Motor shall be suitable for 415 +(-) 10% volts, 50 cycles, 3 phase AC supply. MAKE- EDGETECH, ZECO | Nos | 2 |
| 3 | Air blower for air cooling machines for 15000 CFM CAPACITY | Nos. | 5 |
| 4 | Canvas Connections |  |  |
|  | Supply and fabrication of flexible connections constructed flexible double canvas sleeve in accordance with approved shop drawings and as per the specifications for each packaged unit. | Nos. | 40 |
| 5 | INSULATION AND ACCOUSTICS FOR DUCTS |  |  |
|  | Nitrile Insulation 12mm , UV paint with Fiber cloth and Accoustic (Accoustic lining of duct with 12mm thick resin bonded fiberglass ,fixed with nut and bolts , covered with RP tissue paper and 26 G perforaged aluminum sheet.) | Sqmtr. | 100 |
| 6 | Cooling Pad( Size : 600mm x 200mm x 1800mm) (Make - MUNTERS/HONEYCOMB/Mastercool) | Sqft | 36 |
| 7 | 25mm x 25 mm x 3 mm Mild Steel Angle | Kg | 500 |
| 8 | 38mm x 38 mm x 3 mm Mild Steel Angle | Kg | 500 |
| 9 | 50mm x 50 mm x 6mm Mild Steel Angle | Kg | 500 |

Commercial Bid: (in separate sealed envelope)

Bid for “Supply and installation work for repair and maintenance of Air Cooling Machines”

| LNM INSTITUTE FOR INFORMATION AND TECHNOLOGY,JAMDOLI, JAIPUR |  |  |  |  |
| --- | --- | --- | --- | --- |
| NAME OF WORK : Center Air Cooling Maintenance Work |  |  |  |  |
| PLACE OF WORK : Hostels |  |  |  |  |
| S.no. | DESCRIPTION | Qty. | UOM | AMOUNT |
|  | AIRWASHER MACHINES : |  |  |  |
| 1 | Supply, fabrication, installation and testing of sheet metal ducts in accordance with the approved shop drawing and as per specifications. MAKE -Tata/ESSAR/ JINDAL |  |  |  |
| a | 22 G (0.80mm) galvanized sheet metal steel | 350 | Sqft |  |
| b | 24 G (0.63mm) galvanized sheet metal steel | 256 | Sqft |  |
| c | 22 G galvanized sheet metal steel (with MS flange) | 150 | Sqft |  |
| 2 | 15000 CFM AIRWASHER MACHINES : New body |  |  |  |
|  | Supply,installation, testing & commisioning of double skin package type air washer of minimum 90% efficiency with imported centrifugal fan of static pressure 50 mm of WG, motor, mounting arrangements, metallic vibration isolation, belt drive arrangement, VCD, distribution headers, access door, pump, stainless steel water tank, suction screen, imported cellulose paper fills, prefilter, GI B’ class piping & associated works, quick fill, makeup & drain connections with ball valves & float valves etc complete.Motor shall be suitable for 415 +(-) 10% volts, 50 cycles, 3 phase AC supply. MAKE- EDGETECH, ZECO | 2 | Nos |  |
| 3 | Air blower for air cooling machines for 15000 CFM CAPACITY | 5 | Nos. |  |
| 4 | Canvas Connections |  |  |  |
|  | Supply and fabrication of flexible connections constructed flexible double canvas sleeve in accordance with approved shop drawings and as per the specifications for each packaged unit. | 40 | Nos. |  |
| 5 | INSULATION AND ACCOUSTICS FOR DUCTS |  |  |  |
|  | Nitrile Insulation 12mm , UV paint with Fiber cloth and Accoustic (Accoustic lining of duct with 12mm thick resin bonded fiberglass ,fixed with nut and bolts , covered with RP tissue paper and 26 G perforaged aluminum sheet.) | 100 | Sqmtr. |  |
| 6 | Cooling Pad( Size : 600mm x 200mm x 1800mm) (Make - MUNTERS/HONEYCOMB/Mastercool) | 36 | Sqft |  |
| 7 | 25mm x 25 mm x 3 mm Mild Steel Angle | 500 | Kg |  |
| 8 | 38mm x 38 mm x 3 mm Mild Steel Angle | 500 | Kg |  |
| 9 | 50mm x 50 mm x 6mm Mild Steel Angle | 500 | Kg |  |

| Total |  |
| --- | --- |
| GST |  |
| Total including GST |  |

- Taxes like GST shall be clearly mentioned. If nothing is mentioned, these charges shall be deemed included.

- If the supplier does not have a particular item offered in the tender or a different make is being proposed by him, the same should be highlighted/mentioned especially by him.

- The prices should be including all (Packing, forwarding, Transit Insurance, Transportation Loading/unloading, Installation etc.). Loading/unloading up to 50 Mtr. shall be done by the supplier.

- Bid should be submitted for F.O.R delivery at the destination i.e. The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur- 302031

- Bid should be valid for a minimum period of 30 days. The Change in prices shall not be accepted within the validity period.

- The LNMIIT reserves the right to reject any quotation(s) without assigning any reasons.

- Payment terms shall be decided mutually.

- There should not be any conflict of interest (Declaration to be submitted by the Vendor).

- Bid should be on the company’s letterhead and should submit their bid in the LNMIIT format only

- Item/Service not supplied by the supplier/service provider should be mentioned separately.

- Vendor should provide a photocopy of the cheque/cancelled cheque while presenting the Invoice to LNMIIT.

- Pre-bid meeting and pre-bid site visit can be done before quoting the rate in the tender to clear up any confusion regarding project details, the scope of work, and solicitation of documents.

All charges applicable shall be clearly mentioned with the offer. If nothing is mentioned in the offer, the quoted price shall be treated as final, and nothing extra shall be paid extra.

Signature with seal