The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposal from competent vendors for “Supply of IT Consumables”. Bidders can submit their sealed offer/s at the following address by Speed-Post/ Registered Post/ By-hand (but not by Courier)/ before 05:00 PM on 05.03.2023.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate of the Institute after marking due entries.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Email: directoroffice@lnmiit.ac.in

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory are required to be submitted in two separate envelopes/files. Both envelopes/files should be duly marked as “Technical bid for “Supply of IT Consumables” and “Commercial Bid for “Supply of IT Consumables” respectively. Both Envelopes shall be enclosed in a single envelope bearing the heading Bid for “Supply of IT Consumables”. Bids are liable to be rejected if any or both of the technical or commercial offers are found in open condition or required information is not found with the offer.

Technical Bid: (in separate sealed envelope)

While submitting quotations, the following procedure/ formalities and conditions may kindly be adhered.

- The makers name or brand name must be specified clearly.

- The relevant literature, pamphlets pertaining to the product should be sent along with the tenders / quotations.

- Copy of authorization/certificate from the OEM in case supplied by a vendor who is not a manufacturer of the product.

- Company profile or brief note about the bidder's company, PAN number, GST number should be submitted by the Vendor.

- List of Directors/Partners/Proprietor/Members of the Firm to be attached.

- Detail of other associated/sister concerns ((if any) of Directors/Partners/ Proprietor/ Members to be provided.

- Turn over for last three years to be mention. (Copy to be enclosed)

- List of clients to whom the vendor has delivered the goods or provided the services in past.

- Specification as mentioned below shall be provided as per the details given below:

Technical Bid: (in separate sealed envelope)

Technical specification - Supply of IT Consumables

| Serial Number | Item Name | Quantity | Make/Model | Filled by Vendor |
| --- | --- | --- | --- | --- |
| 01 | Cat6 Cable Box | 02 | Molex |  |
| 02 | External DVD Writer | 02 | HP |  |
| 03 | Fiber Patch Cord Single mode SC-SC (3 Mtrs.) (10G and Higher link Supportable) | 10 | Molex |  |
| 04 | Fiber Patch Cord Single mode SC-LC (3 Mtrs.) (10G and Higher link Supportable) | 10 | Molex |  |
| 05 | Fiber Patch Cord Multimode SC-LC (15 Mtrs.) (10G and Higher link Supportable) | 05 | Molex |  |
| 06 | 8 GB Pen drive | 10 | HP, SanDisk, Kingston |  |
| 07 | RJ - 9 Connector (4 Pin) | 100 | D-Link |  |
| 08 | PVC Insulation Electrical Tape (Black, 1. 8cmx7mx0. 125mm) | 20 | Steel grip |  |
| 09 | Telephone Coil Cord Cable (2 Mtr.) with RJ - 9 Connector | 50 | D-Link, KDL |  |
| 10 | Telephone Line Cord Cable (3 Mtr.) with RJ - 11 Connector | 50 | D-Link, KDL |  |

Commercial Bid: (in separate sealed envelope)

Financial Bid

Supply of IT Consumables

| S/No. | Item Name | Quantity | Amount |
| --- | --- | --- | --- |
| 01 | Cat6 Cable Box | 02 |  |
| 02 | External DVD Writer | 02 |  |
| 03 | Fiber Patch Cord Single mode SC-SC (3 Mtrs.) (10G and Higher link Supportable) | 10 |  |
| 04 | Fiber Patch Cord Single mode SC-LC (3 Mtrs.) (10G and Higher link Supportable) | 10 |  |
| 05 | Fiber Patch Cord Multimode SC-LC (15 Mtrs.) (10G and Higher link Supportable) | 05 |  |
| 06 | 8 GB Pen drive | 10 |  |
| 07 | RJ - 9 Connector (4 Pin) | 100 |  |
| 08 | PVC Insulation Electrical Tape (Black, 1. 8cmx7mx0. 125mm) | 20 |  |
| 09 | Telephone Coil Cord Cable (2 Mtr.) with RJ - 9 Connector | 50 |  |
| 10 | Telephone Line Cord Cable (3 Mtr.) with RJ - 11 Connector | 50 |  |
|  |  | Total |  |
|  |  | Tax |  |
|  |  | Ground Total |  |

- Taxes like GST shall be clearly mentioned. If nothing is mentioned, these charges shall be deemed included.

- If the supplier does not have a particular item offered in the tender or a different make is being proposed by him, the same should be highlighted/mentioned especially by him.

- The prices should be including all (Packing, forwarding, Transit Insurance, Transportation Loading/unloading, Installation etc.). Loading/unloading up to 50 Mtr. shall be done by the supplier.

- Bid should be submitted for F.O.R delivery at the destination i.e. The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur- 302031

- Bid should be valid for a minimum period of 30 days. The Change in prices shall not be accepted within the validity period.

- The LNMIIT reserves the right to reject any quotation(s) without assigning any reasons.

- Payment terms shall be decided mutually.

- There should not be any conflict of interest (Declaration to be submitted by the Vendor).

- Bid should be on the company’s letterhead and should submit their bid in the LNMIIT format only

- Item/Service not supplied by the supplier/service provider should be mentioned separately.

- Vendor should provide a photocopy of the cheque/cancelled cheque while presenting the Invoice to LNMIIT.

- Pre-bid meeting and pre-bid site visit can be done before quoting the rate in the tender to clear up any confusion regarding project details, the scope of work, and solicitation of documents.

All charges applicable shall be clearly mentioned with the offer. If nothing is mentioned in the offer, the quoted price shall be treated as final and nothing extra shall be paid extra.

Signature with seal