TENDER DOCUMENT

FOR

Annual Rate Contact for the supply of  LED Lights

(Tender No.: 2024-25/Estate/2B/LNM-Rate Contract)

THE LNM INSTITUTE OF INFORMATION TECHNOLOGY

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Tender No.: 2024-25/Estate/2B/LNM-Rate Contract

The LNM Institute of Information Technology, Jaipur

TENDER NOTICE

The LNM Institute of Information Technology, Jaipur is a Deemed-to-be-University, set up as a Non Profit Making institute engaged in imparting higher learning in Engineering, Technology and Science. We are located at Gram-Rupa Ki Nangal, Post-Sumel, Via-Jamdoli, Jaipur, Pin 302031 Rajasthan, under the joint venture of the Government of Rajasthan and Lakshmi & Usha Mittal Foundation.

The institute invites proposals from competent vendors for “Annual Rate Contract for the supply of  LED Lights (Item Name)”. Bidders can submit their sealed offer/s at the following address by Email/Speed-Post/ Registered Post/ By-hand/courier before 05:00 PM on 17.05.2024.

Filing of Tender:

Option 1: Physical submission of offers (hard copy) in sealed envelopes.

Bids shall be addressed to the undersigned and deposited in the tender box available at the main gate in person/ Speed-Post/ Registered Post/ By-hand/courier of the Institute after marking due entries in the Receipt Book available with the Security Officer at the main gate.

The tender subject and reference number must be mentioned on the envelope.

Director

The LNM Institute of Information Technology,

Gram – Rupa Ki Nangal, Post – Sumel,

Via – Jamdoli, Jaipur- 302031

Email: directoroffice@lnmiit.ac.in

Option 2: Password-protected online submission via Email

Password-protected offers can be submitted by the bidders via email at the below-mentioned email IDs.

tender-document@lnmiit.ac.in (password-protected document)

tender-password@lnmiit.ac.in (password only)

The tender subject and reference number must be mentioned in the subject line of the emails.

Bid Process:

Technical & Commercial details on the company’s letterhead, duly signed and sealed by the authorized signatory, are required to be submitted in two separate envelopes/emails.

Envelopes/Emails should be duly marked as follows:

- Technical bid for the Annual Rate Contract for the supply of  LED Lights (Item Name)

- Commercial Bid for the Annual Rate Contract for the supply of  LED Lights (Item Name).

Both Envelopes should be enclosed in a single envelope bearing the heading “Bid for Annual Rate Contract for the supply of  LED Lights (Item Name) and 2024-25/Estate/2B/LNM-Rate Contract”.

Bids are liable to be rejected:

- If the above-mentioned bid submission procedure is not followed.

- Incomplete and non-conforming bids.

- Arbitration: All disputes and differences arising between the two parties in connection with this order shall be mutually settled as far as possible, failing which, all such disputes shall be referred to arbitration and settlement under the provision of the Arbitration and Conciliation Act, 1996 and as amended from time to time or any such law for the time being prevailing in India. The venue of such arbitration shall be Jaipur.

Bids can be sent via email as per the process mentioned above.

Technical Bid: (in a separate sealed envelope)

While submitting quotations, the following procedure/ formalities and conditions may kindly be adhered.

- Company profile or brief note about the bidder's company, PAN number, and GST number should be submitted by the Vendor.

- The List of Directors/Partners/Proprietor/Members of the Firm to be attached.

- Detail of other associated/sister concerns (if any) of Directors/Partners/ Proprietor/ Members to be provided.

- Turnover for the last three years to be mentioned. (Copy to be enclosed)

- The Client List with some purchase order and satisfactory certificate (to whom the vendor has delivered the goods/provided the services in the past).

- The Price list and data sheet with the authorization certificate of the OEM should be enclosed.

- All the quoted products should be commercial products with an onsite warranty directly from the supplier. LNMIIT shall not contact the OEM.

- Specification shall be provided as per the details given below:

Commercial Bid: (in separate sealed envelope)

Bid for “Annual Rate Contract for the supply of  LED Light/Fixture (Item Name)”.

| S.No. | Particulars |  | Make (Quoted by the supplier) | Discount on MRP (%) |
| --- | --- | --- | --- | --- |
| 1 | 10 Watt Slim LED Batten Tube Light |  |  |  |
| a | Type | Batten Tube Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 10 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Main Housing | Polycarbonate |  |  |
| g | Front Cover | Polycarbonate Diffuser |  |  |
| h | LED | LM80 Certified LED |  |  |
| i | Ingress protection | IP20 |  |  |
| j | Rated Voltage & Frequency | 240V, 50Hz AC |  |  |
| k | Lumen Maintenance | 50000 Hrs |  |  |
| l | Operating Temperature | 0 C to 50 C |  |  |
| n | Mounting | Surface Mounting |  |  |
| m | Applications | Offices/Other Indoor |  |  |
| 2 | 18 Watt Slim LED Batten Tube Light |  |  |  |
| a | Type | Batten Tube Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 18 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Main Housing | Polycarbonate |  |  |
| g | Front Cover | Polycarbonate Diffuser |  |  |
| h | LED | LM80 Certified LED |  |  |
| i | Ingress protection | IP20 |  |  |
| j | Rated Voltage & Frequency | 240V, 50Hz AC |  |  |
| k | Lumen Maintenance | 50000 Hrs |  |  |
| l | Operating Temperature | 0 C to 50 C |  |  |
| n | Mounting | Surface Mounting |  |  |
| m | Applications | Offices/Other Indoor |  |  |
| 3 | 20 Watt Slim LED Batten Tube Light |  |  |  |
| a | Type | Batten Tube Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 20 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Main Housing | Polycarbonate |  |  |
| g | Front Cover | Polycarbonate Diffuser |  |  |
| h | LED | LM80 Certified LED |  |  |
| i | Ingress protection | IP20 |  |  |
| j | Rated Voltage & Frequency | 240V, 50Hz AC |  |  |
| k | Lumen Maintenance | 50000 Hrs |  |  |
| l | Operating Temperature | 0 C to 50 C |  |  |
| n | Mounting | Surface Mounting |  |  |
| m | Applications | Offices/Other Indoor |  |  |
| 4 | 18 Watt Surface Round Panel Light |  | Nos. |  |
| a | Type | Surface Round Panel Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 18 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Operating Temperature | 0 C to 50 C |  |  |
| g | Rated Voltage /frequency | 240 Voltage /50Hz |  |  |
| h | Size | 225 mm Dia & H : 35mm |  |  |
| i | Main Housing | Excellent thermally managed pressure die cast aluminum |  |  |
| j | Front Cover | Opal Diffuser |  |  |
| k | LED | LM80 certified high power LED |  |  |
| l | Gear | Integral, Electronic, constant current driver |  |  |
| m | Internal Wiring | Industrial grade cooper wire |  |  |
| n | Hardware | SS or Ms zinc plated & passivated |  |  |
| o | Ingress Protection | IP20 |  |  |
| p | Lumen Maintenance | 50000 Hrs |  |  |
| q | Operating Temperature | 0 Degree C to 50 Degree C |  |  |
| r | Mounting | Surface mounted |  |  |
| 5 | 15 Watt Recess Round Panel Light |  |  |  |
| a | Type | Recess Round Panel Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 15 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Operating Temperature | 0 C to 50 C |  |  |
| g | Rated Voltage /frequency | 240 Voltage /50Hz |  |  |
| h | Size | Dia : 200 mm, H : 23 mm, Cutout : 180mm |  |  |
| i | Main Housing | Excellent thermally managed pressure die cast aluminum |  |  |
| j | Front Cover | Polycarbonate Diffuser |  |  |
| k | LED | LM80 certified high power LED |  |  |
| l | Gear | Integral, Electronic, constant current driver |  |  |
| m | Internal Wiring | Industrial grade cooper wire |  |  |
| n | Hardware | SS or Ms zinc plated & passivated |  |  |
| o | Ingress Protection | IP20 |  |  |
| p | Lumen Maintenance | 50000 Hrs |  |  |
| q | Operating Temperature | 0 Degree C to 50 Degree C |  |  |
| r | Mounting | Surface mounted |  |  |
| 6 | 36 Watt 2 x 2 Panel Recess LED Light |  |  |  |
| a | Type | 2 x 2 Tiles Recess mounting |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 36 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Main Housing | Excellent thermally Managed CRCA |  |  |
| g | Front Cover | Polycarbonate Diffuser |  |  |
| h | LED | LM80 certified LED |  |  |
| i | Gear | Integral, Electronic, Constant Current driver |  |  |
| j | Ingress protection | IP20 |  |  |
| k | Rated Voltage & Frequency | 240V, 50Hz AC |  |  |
| l | Lumen Maintenance | 50000 Hrs |  |  |
| m | Mounting ( Size L x W x H ) | Recessed Mounting ( 595 x 595 x 80 MM) |  |  |
| n | Operating Temperature | 0 C to 50 C |  |  |
| 7 | 36 watt 2 x 2 Surface Square Panel LED Light |  |  |  |
| a | Type | 2 x 2 Surface mounting |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 36 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Main Housing | Excellent thermally Managed CRCA |  |  |
| g | Front Cover | Polycarbonate Diffuser |  |  |
| h | LED | LM80 certified LED |  |  |
| i | Gear | Integral, Electronic, Constant Current driver |  |  |
| j | Ingress protection | IP20 |  |  |
| k | Rated Voltage & Frequency | 240V, 50Hz AC |  |  |
| l | Lumen Maintenance | 50000 Hrs |  |  |
| m | Size : L x W x H | 595 mm x 595 mm x 80 mm |  |  |
| n | Operating Temperature | 0 C to 50 C |  |  |
| o | Internal Wiring | Industrial grade cooper wire |  |  |
| p | Hardware | SS or Ms Zinc OR Nickle plated & passivated |  |  |
| 8 | 45 Watt Pole Mounting Round LED Light |  |  |  |
| a | Type | Pole Mounting Round Post Top LED Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 45 Watt |  |  |
| b | Front Cover | Polycarbonate |  |  |
| c | Color Temperature | 5700 K |  |  |
| d | Operating Temperature | 0 C to 50 C |  |  |
| f | Rated Voltage & Frequency | 240V, 50Hz AC |  |  |
| i | Size | O : 400 mm Dia & H : 263mm |  |  |
| j | Main Housing | Pressure Die cast aluminum |  |  |
| k | LED | LM80 certified LED |  |  |
| l | Gear | Integral, Electronic, constant current driver |  |  |
| m | Internal Wiring | Industrial grade cooper wire |  |  |
| n | Hardware | SS or Ms zinc plated & passivated |  |  |
| o | Ingress Protection | IP65 |  |  |
| p | Lumen Maintenance | 50000 Hrs |  |  |
| q | Operating Temperature | 0 Degree C to 50 Degree C |  |  |
| 9 | 45 Watt External photo Sensor for Automatic ON/OFF LED Street Lights |  |  |  |
| a | Type | Pole Mounting Street LED Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 45 Watt |  |  |
| b | Rated Voltage / Rated frequency | 240 Voltage /50Hz |  |  |
| c | Colour Temperature | 5700 K |  |  |
| d | Main Housing | Excellent thermally managed pressure die cast aluminum |  |  |
| f | Front Cover | Integrated PC Lens |  |  |
| i | LED | LM80 certified LED |  |  |
| j | Lens | Batwing 135/85 Degree |  |  |
| k | Sensor | External Photo sensor for automatic on/off, Sensor mounting outside the luminaires with bracket. |  |  |
| l | Gear | Integral, Electronic, constant current driver |  |  |
| m | Internal Wiring | Industrial grade cooper wire |  |  |
| n | Hardware | SS or Ms zinc plated & passivated |  |  |
| o | Ingress Protection | IP66 |  |  |
| p | Lumen Maintenance | 50000 Hrs |  |  |
| q | Operating Temperature | 0 Degree C to 50 Degree C |  |  |
| 10 | 36 Watt External photo Sensor for Automatic ON/OFF LED Street Lights |  |  |  |
| a | Type | Pole Mounting Street LED Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 36 Watt |  |  |
| b | Rated Voltage / Rated frequency | 240 Voltage /50Hz |  |  |
| c | Colour Temperature | 5700 K |  |  |
| d | Main Housing | Excellent thermally managed pressure die cast aluminum |  |  |
| f | Front Cover | Integrated PC Lens |  |  |
| i | LED | LM80 certified LED |  |  |
| j | Lens | Batwing 135/85 Degree |  |  |
| k | Sensor | External Photo sensor for automatic on/off, Sensor mounting outside the luminaires with bracket. |  |  |
| l | Gear | Integral, Electronic, constant current driver |  |  |
| m | Internal Wiring | Industrial grade cooper wire |  |  |
| n | Hardware | SS or Ms zinc plated & passivated |  |  |
| o | Ingress Protection | IP66 |  |  |
| p | Lumen Maintenance | 50000 Hrs |  |  |
| q | Operating Temperature | 0 Degree C to 50 Degree C |  |  |
| 11 | 200 Watt LED Flood Light |  |  |  |
| a | Type of Light | LED Flood Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 5 years |  |  |
| d | Rated power (Watt ) | 200 Watt |  |  |
| b | Rated Voltage / Rated frequency | 240 Voltage /50Hz |  |  |
| c | Size (L x W x H ) | 372 x 324 mm |  |  |
| d | Colour Temperature | 5700 K |  |  |
| f | Main Housing | Excellent thermally managed pressure die cast aluminum |  |  |
| i | Front Cover | Clear Toughened Glass |  |  |
| j | LED | LM80 certified high power LED |  |  |
| k | Lens | Secondary Optics 60 Degree |  |  |
| l | Gear | Integral, Electronic, constant current driver |  |  |
| m | Internal Wiring | Industrial grade cooper wire |  |  |
| n | Hardware | SS or Ms zinc plated & passivated |  |  |
| o | Ingress Protection | IP66 |  |  |
| p | Lumen Maintenance | 50000 Hrs |  |  |
| q | Operating Temperature | 0 Degree C to 50 Degree C |  |  |
| r | Mounting | Bracket provided for mounting |  |  |
| 12 | 50 Watt LED Flood Light |  |  |  |
| a | Type of Light | LED Flood Light |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 5 years |  |  |
| d | Rated power (Watt ) | 50 Watt |  |  |
| b | Rated Voltage / Rated frequency | 240 Voltage /50Hz |  |  |
| c | Size (L x W ) | 125 x 269 mm |  |  |
| d | Colour Temperature | 5700 K |  |  |
| f | Main Housing | Excellent thermally managed pressure die cast aluminum |  |  |
| i | Front Cover | Clear Toughned Glass |  |  |
| j | LED | LM80 certified high power LED |  |  |
| k | Lens | Secondary Optics 60 Degree |  |  |
| l | Gear | Integral, Electronic, constant current driver |  |  |
| m | Internal Wiring | Industrial grade cooper wire |  |  |
| n | Hardware | SS or Ms zinc plated & passivated |  |  |
| o | Ingress Protection | IP66 |  |  |
| p | Lumen Maintenance | 50000 Hrs |  |  |
| q | Operating Temperature | 0 Degree C to 50 Degree C |  |  |
| r | Mounting | Bracket provided for mounting |  |  |
| 13 | 24 watt 1 x 1 Recess Square Panel LED Light |  |  |  |
| a | Type | 1 x 1 Recess Square mounting |  |  |
| b | Brand/ Make | Orient/Philips/CG |  |  |
| c | Warranty | 2 years |  |  |
| d | Rated power (Watt ) | 24 Watt |  |  |
| e | Color Temperature | 6500 K |  |  |
| f | Main Housing | Excellent thermally Managed pressure Die Cast Aluminium |  |  |
| g | Front Cover | Opal Diffuser |  |  |
| h | LED | LM80 certified LED |  |  |
| i | Gear | Integral, Electronic, Constant Current driver |  |  |
| j | Ingress protection | IP20 |  |  |
| k | Rated Voltage & Frequency | 240V, 50Hz AC |  |  |
| l | Lumen Maintenance | 50000 Hrs |  |  |
| m | Size : L x W x H | 300 mm x 300 mm x 13 mm |  |  |
| n | Operating Temperature | 0 C to 50 C |  |  |
| o | Internal Wiring | Industrial grade cooper wire |  |  |
| p | Hardware | SS or Ms Zinc OR Nickle plated & passivated |  |  |

- Financial Bid-2 for price quotations shall be opened in respect of those tenderer who qualify Technical Bid-1 for specifications and other requirements.

- Tender should be signed by the authorized person.

- Tenders containing false, incomplete and/ or inadequate information are liable to be rejected.

- Conditional tender shall not be accepted. If the supplier does not have a particular item offered in the tender or a different make is being proposed by him, the same should be highlighted/mentioned especially by him.

- The prices should include all (Packing, forwarding, Transit Insurance, Transportation Loading/unloading, Installation etc.). Loading/unloading up to 50 Mtr. shall be done by the supplier.

- Bid should be submitted for F.O.R delivery at the destination i.e. The LNM Institute of Information Technology, Gram – Rupa Ki Nangal, Post – Sumel, Via – Jamdoli, Jaipur- 302031

- The bid should be valid for a minimum period of one year (it can be extended on yearly basis with mutual consent). The Change in prices shall not be accepted within the validity period.

- The LNMIIT reserves the right to reject any quotation(s) without assigning any reasons.

- Payment terms shall be decided mutually.

- There should not be any conflict of interest (Declaration to be submitted by the Vendor).

- The bid should be on the company’s letterhead and should submit their bid in the LNMIIT format only.

- Item/Service not supplied by the supplier/service provider should be mentioned separately.

- Vendor should provide a photocopy of the cheque/cancelled cheque while presenting the Invoice to LNMIIT.

- Pre-bid meeting and pre-bid site visit can be done before quoting the rate in the tender to clear up any confusion regarding project details, the scope of work, and solicitation of documents.

- If any vendor is applying for the first time in the LNMIIT, Jaipur then they must submit their last three years working/ supplying record in the work domain.

- Vendors must submit their bids in the LNMIIT format other than that given format shall be rejected.

Signature and Seal of the Tenderer

With the address of the Insurance Company